Innovations to Support Instream Flow Determinations in Texas, USA

II:  Stream Classification System

Presenter:  Wendy Gordon, Texas Commission on Environmental Quality

Abstract An integrated stream classification system was developed and tested for Texas based on quantitative data for 18 distinguishing parameters encompassing watershed and stream channel processes and functions from four disciplines: (1) Hydrology & Hydraulics, (2) Water Quality, (3) Geomorphology & Physical Processes, and (4) Climatology.  The State of Texas was partitioned into five regions: East Texas, South-Central Texas, Lower Rio Grande Basin, West Texas, and North-Central Texas by 8-digit Hydrologic Unit Code (HUC) basin.

Person Completing this Form Wendy Gordon, Ph.D., Texas Parks and Wildlife Department

Today’s Date 04/17/2008

Project Date and Duration 9/1/2006 to 8/31/2007

Project Collaborators Eric Hersh and David Maidment, Ph.D., Center for Research in Water Resources, The University of Texas at Austin

Project Location Texas, USA

Study Area (km2) 695621

Aim/Purpose This stream classification system might be used to: (1) discern likely similarities and differences between rivers and streams of the State, (2) remotely characterize stream segments for which resources are insufficient for detailed field studies, (3) recognize streams and watersheds of the State as having common identities, (4) allow conclusions drawn from an instream flow study from a particular river reach to have a wider applicability than the particular study site, and (5) assist in prioritization of rivers and reaches for future instream flow studies.

Objectives Use Geographic Information System (GIS) technology to organize existing information relevant to the understanding of Texas streams and rivers (i.e., water quality, geologic and geomorphic, hydrologic and hydraulic, and biological data) and to develop a classification scheme such that particular classes or regions of streams and rivers could be recognized as having a common identity.

Specific Activities and Methods Used an existing, qualitative scheme of Texas rivers and streams presented in the National Research Council’s review of the Texas Instream Flow Program as the basis of the project.  Incorporated existing GIS-based data such as the National Hydrography Dataset and other data layers.  Used eight-digit HUCs as the base layer for classification. Quantitative information such as climatology, soil type, water quality monitoring data were added to refine the qualitative classification scheme and produce a more quantitative scheme.

Data Used See report at http://www.crwr.utexas.edu/reports/pdf/2007/rpt07-02.pdf

Data Generated Classification scheme, data interpreted to the HUC level, and geodatabases

Results/Outcomes A more quantitatively-based classification scheme than we started with that can be further refined with the addition of biological data in particular.

Products Geodatabases and maps

Management and Policy Application Texas is too large a state to conduct detailed instream flow studies for all rivers and streams.  Thus, developing classification schemes is one way to generalize or leverage the information we do have about instream flow needs across stream types or classes.  This project was one of two undertaken to aid in that generalization process.  Ideally, relationships between aquatic biota and hydrology and other physical parameters would be developed in order to fully operationalize the classification schemes for instream-flow protection.

Monitoring NA

Funding $75,000 USD

Funding Source US EPA Clean Water Act Section 106 pass through dollars to agency

Documents Hersh, Eric S. and Maidment, David R., 2007, An integrated stream classification system for Texas, Center for Research in Water Resources Online Report 07-02, 130 p., http://www.crwr.utexas.edu/reports/pdf/2007/rpt07-02.pdf