Marine Ecological Gap Analysis Methodology for the

Darwin Initiative to enhance an established marine protected area system in the Cayman Islands

A Darwin Initiative Project funded by the Department for Environment, Food and Rural Affairs (DEFRA), UK.

August 2012

Jeremy Olynik, Laura Richardson, Steve Schill

Marine Ecological Gap Analysis methodology for the Darwin Initiative to enhance an established marine protected area system in the Cayman Islands

Jeremy Olynik1, Laura Richardson1,2 and Steve Schill3

1 The Cayman Islands Government Department of Environment; 2 Bangor University School of Ocean Sciences, Wales; and 3 The Nature Conservancy, USA.

Darwin Initiative Project reference: 	18-016

Citation:	Olynik, J, Richardson, L, and Schill S. 2012. Marine Ecological Gap Analysis methods for the Darwin Initiative to enhance an established marine protected area system in the Cayman Islands. Report prepared by The Cayman Islands Department of Environment, Bangor University and The Nature Conservancy, 41pp.

More information on the Darwin Initiative project and its collaborative partners can be found at:

darwin.defra.gov.uk/project/18016

www.DOE.ky

www.sos.bangor.ac.uk/

www.nature.org

Copyright © 2013 by the Cayman Islands Government. All text, designs, graphics and other works in this document are the copyrighted works of the Cayman Islands Government. All rights reserved. Any redistribution, in whole or in part, without the permission of the Cayman Islands Government, is strictly prohibited.  Reproduction of material in this information product for resale or other commercial purposes is prohibited without written permission of the copyright holders. Applications for such permission should be addressed to:

Department of Environment

Tel. +1-345-9498469, Fax. +1-345-9494020

DOE@gov.ky

Cover photo: Tiger grouper in Grand Cayman. © Croy McCoy.

Disclaimer: This document is made possible by the support of the United Kingdom’s Department of Environment, Food and Rural Affairs (DEFRA). The contents are the responsibility of the Darwin Initiative, the Cayman Islands Department of Environment (DOE), Bangor University, and The Nature Conservancy (TNC) and do not necessarily reflect the views of DEFRA or the Cayman Islands Government.

Contents

Acronyms	1

Executive summary	2

Acknowledgements	3

1.	Background	4

2.	Marxan conservation planning software	4

3.	Study area	5

4.	Selection of strata	6

5.	Development of Environmental Risk Surface models	9

5.1	Terrestrial flow accumulation to model marine environment risk from surface runoff	10

5.1.1	Grand Cayman	11

5.1.2	Little Cayman	12

5.1.3	Cayman Brac	14

5.2	Defining the marine Environmental Risk Surfaces	16

5.2.1	Grand Cayman	16

5.2.2	Little Cayman	17

5.2.3	Cayman Brac	18

6.	Conservation targets definition and goal setting	19

7.	Design of planning units	21

8.	Inclusion of status layers	22

9.	Marxan results	24

10.	Discussion	26

10.1	Reef Health Index	27

10.2	Fishing pressure location and intensity	28

10.3	Next Steps	29

11.	References	29

List of Tables

Table 1 Final strata selected for each of the Cayman Islands.	6

Table 2 Grand Cayman terrestrial risk elements.	11

Table 3 Little Cayman terrestrial risk elements.	12

Table 4 Terrestrial risk elements for Cayman Brac.	14

Table 5 Marine risk elements for Grand Cayman.	16

Table 6 Marine risk elements for Little Cayman.	17

Table 7 Marine risk elements for Cayman Brac.	18

Table 8 Marine conservation targets for all three Cayman Islands	21

List of Figures

Figure 1 Development of Ecological Gap Analysis using Marxan (Schill and Raber, 2011).	5

Figure 2 The Cayman Islands of Grand Cayman, Little Cayman and Cayman Brac and their position in the Caribbean.	6

Figure 3 Final strata for all Cayman Islands, covering areas extending from the natural reef ‘drop off’ (approximately 80 ft depth) up to 200 ft inland of the high-watermark. The outer limit of strata was the 650 ft depth contour where spawning aggregation sites exist.	9

Figure 4 Examples of distance decay functions and the resulting output based on a single point occurrence and a 1000m influence distance. Red and orange hues indicate higher intensity values while green and blue hues represent lower intensity values.	10

Figure 5 Terrestrial Environmental Risk Surface (ERS) for Grand Cayman. Highlights the level of risk to the terrestrial environment and includes the intensity, influence distance, distance decay, and overlay function. This information was combined with the flow accumulation model so that coastal outlet points could be assigned a level of risk to the marine environment due to surface run-off.	11

Figure 7 Terrestrial Environmental Risk Surface (ERS) for Little Cayman. Highlights the level of risk to the terrestrial environment and includes the intensity, influence distance, distance decay, and overlay function. This information was combined with the flow accumulation model so that coastal outlet points could be assigned a level of risk to the marine environment due to surface run-off.	13

Figure 9 Terrestrial Environmental Risk Surface (ERS) for Cayman Brac. Highlights the level of risk to the terrestrial environment and includes the intensity, influence distance, distance decay, and overlay function. This information was combined with the flow accumulation model so that coastal outlet points could be assigned a level of risk to the marine environment due to surface run-off.	15

Figure 11 Marine Environmental Risk Surface (ERS) for Grand Cayman. Highlights the risks to the marine environment and includes their intensity, influence distance, distance decay, and overlay function.	17

Figure 12 Marine Environmental Risk Surface (ERS) for Little Cayman. Highlights the risks to the marine environment and includes their intensity, influence distance, distance decay, and overlay function.	18

Figure 13 Marine Environmental Risk Surface (ERS) for Cayman Brac. Highlights the risks to the marine environment and includes their intensity, influence distance, distance decay, and overlay function.	19

Figure 14 Conservation features are overlaid on top of risks and each planning unit (1 acre) is assigned a score based on the level of cost associated with obtaining a conservation feature within each unit. Conservation feature layer has been clipped for display purposes but should match the extents of the risk surface and planning units.	22

Figure 15‘No-Take Protected Areas’ Strict status layers for all Cayman Islands – planning units within these areas are first considered when attempting to meet conservation goals. ‘All Protected Areas’ and ‘NO Protected Areas’ status layers were also considered in the final Marxan output scenarios.	24

Figure 16 Modeled Marxan scenarios for all Cayman Islands’ marine environment. The maps display a scenario where the ERS is included, ‘No-Take Marine Parks’ Strict status layers are locked in, 30% and 40% (Base) protection goals are in place, a BLM of 150, and a constant SPF value of 1 for all conservation targets used.	26

Figure 17 Threshold values by indicator for the Simplified Integrated Reef Health Index (SIHRI). Source: Healthy Reefs Initiative (2010) Report Card for the Mesoamerican Reef.	27

Figure 18 Reef health index (Simplified Integrated Reef Health Index), combining coral cover, fleshy macro-algal cover, herbivorous fish abundance and commercial fish abundance at 63 permanent sampling sites; www.healthyreefs.org. (Regional threshold values: 1=Critical, 2=Poor, 3=Fair, 4=Good, 5=Very good).	28

Figure 19 Spatial distribution of fishing effort over a monthly period on all Cayman Islands. Star symbols show locations of the major boat launch areas.	29

List of Appendices

Appendix 1 Final list of Conservation Targets.	31

Appendix 2 Final output file of Marxan best scenario for ‘base goals’ and strict status layers locked in.	35

Appendix 3 Data (2011) used for the Simplified Integrated Reef Health Index for the Cayman Islands.	41

# Acronyms

BAP		Biodiversity Action Plan

BLM		Boundary Length Modifier

CUC		Caribbean Utilities Company

DEFRA	Department for Environment, Food and Rural Affairs

DEM		Digital Elevation Model

DOE		Department of Environment

DST		Decision Support Tool

ERS		Environmental Risk Surface

ESRI		Environmental Systems Research Institute

GIS		Geographic Information System

ID		Identification

IRHI		Integrated Reef Health Index

LIDAR		Light Detection and Ranging

MARXAN	acronym combining MARine, and SPEXAN, itself an acronym for SPatially EXplicit ANnealing

MPA		Marine Protected Area

MPM		Minimum Proportion of targets Met

PAT		Protected Area Tool

SIHRI		Simplified Integrated Reef Health Index

SMB		Seven Mile Beach

SOS		School of Ocean Sciences

SPAG		Spawning Aggregation

TNC		The Nature Conservancy

UK		United Kingdom

USA		United States of America

ZC		Zonae Cogito

# Executive summary

The Cayman Islands Government Department of Environment (DOE), in collaboration with Bangor University, Wales (School of Ocean Sciences – SOS) and The Nature Conservancy, USA (TNC), has conducted a review of the current Marine Protected Areas (MPAs) of the Cayman Islands to determine whether they are optimal in size and area, appropriately located, and provide maximum resilience.  The necessary enhancements to this system of protected areas have been subsequently recommended to Cabinet for their approval. This report outlines the methods and results of the Marine Ecological Gap Analysis utilizing Marxan conservation planning software which has been a key element of this review.

Marxan is a computer software program that provides decision support to a range of conservation planning. It calculates an optimal conservation solution, or portfolio, based on two core data inputs. The first input is a set of conservation features or ‘targets’; the second are direct and indirect anthropogenic risks to the marine environment from both marine and terrestrial sources.

The study area for this project encompassed the marine and terrestrial environment for all three Cayman Islands; Grand Cayman, Little Cayman, and Cayman Brac. The effective study area ranged from 200 ft inland of the high-water mark to an outer edge of approximately 80 ft depth below mean sea level. Further, the spatial extent of the Cayman Islands was divided into strata. The division of the islands by strata was designed to disperse portfolio selection across the extent of the study area.

The first step was to create a marine Environmental Risk Surface (ERS) for each island which is a modelled Geographic Information System surface developed using mapped risk elements. A ‘risk element’ can be defined as anything identified by experts as having a negative influence on the health of a critical habitat or key species. Included in the marine ERSs were negative effects on the marine environment due to terrestrial risks utilizing a flow accumulation model.

The next step was to compile a list of conservation targets for each island and create spatial layers. Broad categories were created to group target candidates into the general classes of: 1) Outside Lagoon Benthic 2) Lagoon Benthic, 3) Shoreline 4) Wetland 5) Protected Land 6) Proposed Land For Protection and 7) Species.

Goals for percentage of protection for each conservation target were then compiled. In the ‘Base Goals’ scenario solution, those targets considered most important were given a goal of 40% protection of their total area and less important targets were assigned 30% of their total area within each strata. Targets of high importance were determined by their ability to support a high level of biodiversity as well as their importance to key species. Additionally, high goals were also included in the table in order to generate an ‘ultra-conservative’ scenario where Marxan was tasked with trying to protect 80% of the representation of the most important targets.

Planning units were then defined by overlaying the planning region with a lattice of 120,122 one acre (0.405 ha) hexagons. Planning units allow for the overlay of risk elements and conservation targets in an effort to determine the most efficient distribution of conservation areas. Three scenarios were examined for modelled outputs. The first was a scenario where all current Marine Parks and special interest areas (or ‘status layers’) were locked-in, thus always included in the solution. The second scenario was one where only current no-take Marine Parks were locked-in. The third option did not lock-in any existing Marine Parks so that all areas had equal chance of being included in the modelled scenario.

The Protected Area Tools (PAT) extension for ArcGIS 9.3 was utilized to create input files that fed into Zonae Cogito (ZC) decision support software. ZC combines planning units, strata, targets, risks, goals, and status layers to run and visualize model outputs.

Following the Marxan modelling exercise, additional Decision Support Tools (DSTs) were utilized to assist in determining the best Marine Park design using as much relevant information as possible. Guided by the generated Marxan solutions, complementary DSTs, and extensive public consultation feedback the DOE and partners drafted recommendations for an enhanced system of Marine Protected Areas for the Cayman Islands.

# Acknowledgements

Persons who have provided inputs, assisted with review, and were main contributors to the project were DOE personnel: Gina Ebanks-Petrie, Timothy Austin, Croy McCoy, John Bothwell, Bradley Johnson, Phil Bush, and Gene Parsons. All Department of Environment staff have been involved at certain stages of the project and have given invaluable advice and discussion. James Byrne (TNC) and John Turner (SOS) were also a tremendous help in designing the final solutions and Decision Support Tools.

# Background

Between April 2010 to March 2013 the Cayman Islands Government Department of Environment (DOE) collaborated with international partners, Bangor University, Wales (School of Ocean Sciences – SOS) and The Nature Conservancy, USA (TNC) on Darwin Initiative project, Darwin Initiative to Enhance an Established Marine Protected Area System (DEFRA reference 18-016). The project proposed to review the Marine Protected Areas (MPAs) of the Cayman Islands which were established in 1986, to determine whether they are optimal in size and area, appropriately located, and provide maximum resilience and enhance them as necessary.

The work built upon the results of a previous Darwin Initiative project In Ivan’s Wake - Darwin Initiative BAP for the Cayman Islands, Government of Cayman Islands and University of Exeter in Cornwall, whose principle output was a National Biodiversity Action Plan for the Cayman Islands. Detailed habitat mapping of the marine and terrestrial environments in this previous project contributed significantly to the ecological gap analysis carried out in the project described herein.

Collaborative relations existed between the DOE and TNC prior to Darwin Initiative project 18-016, with  TNC providing an Ecological Gap Analysis/Marxan training workshop to DOE personnel in 2009.  Since commencement of the project, the TNC has continued this training and provided expertise in the construction and implementation of the Cayman Islands ecological gap analysis for the purpose of utilizing science-based systematic conservation planning methods to strategically expand the Cayman Islands MPA network.

The project objectives were as follows:

- To assess the current level of reef resilience within and outside all marine protected areas of Grand Cayman, Little Cayman and Cayman Brac;

- To examine representativeness and variation by utilizing mapped reef and associated ecosystem habitats;

- To assess overspill of fish biomass from all No-Take zones;

- To quantify the impact of the artisanal/recreational fishery;

- To use the data from 1-4 to plan and promote an extension to the MPA system with full public consultation and involvement from stakeholders (represented by the Marine Conservation Board/Watersports Association/District Communities).

The purpose of the current report is to outline the methods and results of the Marine Ecological Gap Analysis (http://www.cbd.int/protected-old/gap.shtml) utilizing Marxan conservation software (Ball et al, 2009), completed by DOE since 2008 and in fulfillment of Objective 2 listed above.

# Marxan conservation planning software

Marxan (acronym combining MARine, and SPEXAN, itself an acronym for SPatially EXplicit ANnealing) is a computer software program designed to aid systematic reserve design which provides decision support to a range of conservation planning challenges including; the design of new marine reserve systems based on pre-determined goals, reporting on the performance of existing reserve systems, and developing multiple-use zone plans for natural resource management. It is used as part of a systematic conservation planning process. Marxan is currently the most widely used conservation planning software in the world (http://www.uq.edu.au/marxan/).

Marxan calculates an optimal conservation solution, or portfolio, based on two core data inputs. The first input is a set of conservation features (habitat types or areas that need protection) for the marine environment such as coral reefs, seagrass beds, critical habitat of key species, etc. The second main input is risk to the marine environment such as SCUBA diving intensity, sources of pollution, dredged seabed, boat traffic intensity, etc. Marxan is efficient and repeatable, providing a variety of solution options which can then be reviewed and analyzed by resource managers and through stakeholder participation. The following flowchart describes the basic steps taken in developing a marine Ecological Gap Analysis for the Cayman Islands using Marxan (Figure 1).

Figure 1 Development of Ecological Gap Analysis using Marxan (Schill and Raber, 2011).

# Study area

The study area for Marxan mapping encompassed the marine and terrestrial environment for all three Cayman Islands (Figure 2 The Cayman Islands of Grand Cayman, Little Cayman and Cayman Brac and their position in the Caribbean. extending from the reef shelf ‘drop-off’ (the limit of benthic habitat mapping,approximately 80 ft depth) up to 200 ft inland of the high-watermark. Where known spawning aggregation sites (SPAGs) exist on each island, benthic mapping area coverage extended offshore to the 650 ft depth contour in these areas. The area 200 ft inland of the high-watermark was included so that coastal mangrove areas could be added as a conservation target due to their high importance to the marine environment. Further, coastal protected land and proposed protected land were also included as targets or ‘conservation features’.

Figure 2 The Cayman Islands of Grand Cayman, Little Cayman and Cayman Brac and their position in the Caribbean (inset).

# Selection of strata

Strata,or stratification, is the classification of data into subcategories on the basis of chosen criteria. For the current project, the spatial extent of the Cayman Islands was divided into geographic strata. Division of the islands in this way facilitated the dispersal of portfolio selection across the extent of the study area (outlined above) in order to ensure conservation efforts are widely spread, thereby minimizing the risk that all examples of a habitat will be damaged by the same disturbance (such as a hurricane, oil spill, boat grounding etc.). Stratification also represented areas of unique ecological significance (such as SPAGs or important nursery habitats).

Grand Cayman was divided into five final marine strata; Little Cayman into two; and Cayman Brac also into two strata (Figure 3). Each strata was given a unique ID so that it could be linked to conservation features within each area and then entered into Marxan (Table 1).

Table 1 Final strata selected for each of the Cayman Islands.

| Island | Strata Description | Strata ID |
| --- | --- | --- |
| Grand Cayman | North Sound | 10 |
| Grand Cayman | West Side | 20 |
| Grand Cayman | South Side | 30 |
| Grand Cayman | North Side | 40 |
| Grand Cayman | East End | 50 |
| Little Cayman | Little Cayman North | 60 |
| Little Cayman | Little Cayman South | 70 |
| Cayman Brac | Cayman Brac North | 80 |
| Cayman Brac | Cayman Brac South | 90 |

Figure 3 Final strata for all Cayman Islands, covering areas extending from the natural reef ‘drop off’ (approximately 80 ft depth) up to 200 ft inland of the high-watermark. The outer limit of strata was the 650ft depth contour where spawning aggregation sites exist.

# Development of Environmental Risk Surface models

An Environmental Risk Surface (ERS), or Cost Surface, is a modeled surface developed using mapped risk elements (e.g. socioeconomic information, human activity, etc.) to explore the overlap between these risk elements and biodiversity features (Ball et al. 2009, Schill and Raber, 2011). A ‘risk element’ can be defined as anything identified by experts as having a negative influence on the health of a critical habitat or key species. The ERS measures cumulative levels of risk impacts across the land or seascape and can be used to focus conservation site selection by steering habitat selection away from high-risk areas where the abatement of pressures on biodiversity seems less likely. An essential part of the ERS process is that risks must be based on features that may be spatially mapped.  For this project, a large volume of spatial data was created based on expert opinion which was then used to create robust ERS models for each Island.

Part of the ERS development included listing and ranking all possible risk elements and assigning model variables including intensity, influence distance, distance decay function, and element overlap function (Figure 4) (Schill and Raber, 2009).

Intensity - degree of risk to the focal habitat/species (set on a relative scale) ranked and scored by expert consensus based on the relation to other risk elements. The intensity scores are not a representation of the absolute measure of the impact of activities on biodiversity. Rather, these values are a relative degree to which biodiversity is more likely to survive in one place over another based on the presence of a given activity in comparison to another activity.

Influence distance - the spatial extent or footprint of the risk element that represents the maximum distance of negative impact on a conservation target. In other words, as the distance of the buffer increases away from the center (point, line or polygon) where the activity is taking place, the intensity values of the cells within the buffers diminish progressively (distance decay) and the risk to the target lessens.

Distance decay – indicates how the intensity of a risk element decreases within the range of the influence distance. There are four types for expressing the decay function: a) linear – decreases linearly; b) concave – decreases rapidly; c) convex – decreases slowly; and d) constant – constant decrease. For all four types, when the maximum distance is reached, the intensity becomes zero.

Overlay function - how overlaying areas of influence distances between neighboring risk element features are resolved using map algebra statistical functions.

Figure 4 Examples of distance decay functions and the resulting output based on a single point occurrence and a 1000 m influence distance. Red and orange hues indicate higher intensity values while green and blue hues represent lower intensity values.

## Terrestrial flow accumulation to model marine environment risk from surface runoff

Included in the marine ERSs created were potential terrestrial risks to the marine environment such as landfills, roadways, invasive species, etc. (which were mapped and assigned model variables during the February 2009 meeting between DOE and TNC; for example see Figure 5). These elements were used as input (weighting) to a flow accumulation model to better understand their impacts on marine habitats at each coastal outlet. This function requires a flow direction grid from a Digital Elevation Model (DEM). The flow direction model indicates the direction water flows on a cell-by-cell basis, starting at higher elevations and moving downward to the coastal outlet, simulating water moving across a sloped surface. The intensity values of the terrestrial risk elements are accumulated on a cell-by-cell basis as the model runs from the higher elevations to the coastal outlet. The final accumulated grid surface calculates accumulated risk (in intensity values) upstream from any outlet which is an indication of potential upland impact on coastal environments. GIS point features are then created for each coastal outlet and the accumulated intensity values are extracted and transferred to the point attribute table. The intensity values attached to these coastal points are evaluated and assigned a classification based on the accumulated values (very high, high, medium, low) and corresponding intensity and influence distance values (listed in the risk Table 2-4 below).

To create flow accumulation models we used Light Detection and Ranging (LIDAR) elevation data resampled to a 10 ft cell size, which was the same cell size used for the ERS models. All terrestrial ERS models were rescaled to 0-1 to constrain the accumulated values. Comprehensive values used in the calculation of the environmental risk surfaces for each of the islands are displayed in Table 2-4 with maps of model results for both the terrestrial risks and modeled flow accumulation of intensity values (Figure 5-10).

### Grand Cayman

The terrestrial ERS risks determined for Grand Cayman (and used for weighting of flow accumulation risks to the marine environment) were as follows:

Table 2 Grand Cayman terrestrial risk elements.

| Risk | Class | Intensity | Influence distance | Decay function | Overlay function |
| --- | --- | --- | --- | --- | --- |
| Landfill | N/A | 95 | 300 ft (91 m) | Linear | Maximum |
| Buildings | Very high | 80 | 300 ft (91 m) | Linear | Maximum |
| Buildings | High | 70 | 50 ft (15 m) | Linear | Maximum |
| Airports | N/A | 70 | 300 ft (91 m) | Linear | Maximum |
| Ports | N/A | 70 | 300 ft (91 m) | Linear | Maximum |
| Quarries | Wet | 60 | 100 ft (30 m) | Linear | Maximum |
| Quarries | Dry | 60 | 300 ft (91 m) | Linear | Maximum |
| Buildings | Low | 60 | 25 ft (8 m) | Linear | Maximum |
| Roads | Primary | 50 | 50 ft (15 m) | Linear | Maximum |
| Roads | Secondary | 30 | 40 ft (12 m) | Linear | Maximum |
| Golf course | N/A | 20 | 25 ft (8 m) | Linear | Maximum |
| Roads | Unpaved | 15 | 25 ft (8 m) | Linear | Maximum |
| Man-modified | N/A | 15 | 25 ft (8 m) | Linear | Maximum |
| MRCU canals | N/A | 10 | 0 | Concave | Maximum |
| Invasives | Casuarina | 5 | 10 ft (3 m) | Concave | Maximum |
| Invasives | Scaevola | 5 | 10 ft (3 m) | Concave | Maximum |

Figure 5 Terrestrial Environmental Risk Surface (ERS) for Grand Cayman. Highlights the level of risk to the terrestrial environment and includes the intensity, influence distance, distance decay, and overlay function. This information was combined with the flow accumulation model so that coastal outlet points could be assigned a level of risk to the marine environment due to surface run-off.

Figure 6 Simplified version of the flow accumulation model and coastal outlet points (pour points) for Grand Cayman. The model highlights the major flow of water across the surface of the island, as well as the major coastal outlets, based on LIDAR elevation data.

### Little Cayman

The terrestrial ERS risks determined for Little Cayman were as follows:

Table 3 Little Cayman terrestrial risk elements.

| Risk | Class | Intensity | Influence distance | Decay function | Overlay function |
| --- | --- | --- | --- | --- | --- |
| Landfill | N/A | 95 | 300 ft (91 m) | Linear | Maximum |
| Buildings | High | 70 | 50 ft (15 m) | Linear | Maximum |
| Airports | N/A | 70 | 300 ft (91 m) | Linear | Maximum |
| Salt rocks dock | N/A | 70 | 300 ft (91 m) | Linear | Maximum |
| Quarries | Dry | 60 | 300 ft (91 m) | Linear | Maximum |
| Buildings | Low | 60 | 30 ft (9 m) | Linear | Maximum |
| Roads | Primary | 50 | 50 ft (15 m) | Linear | Maximum |
| Roads | Secondary | 30 | 40 ft (12 m) | Linear | Maximum |
| Roads | Unpaved | 15 | 30 ft (9 m) | Linear | Maximum |
| Man-modified | N/A | 15 | 30 ft (9 m) | Linear | Maximum |
| Invasives | Casuarina | 5 | 10 ft (3 m) | Concave | Maximum |
| Invasives | Scaevola | 5 | 10 ft (3 m) | Concave | Maximum |

Figure 7 Terrestrial Environmental Risk Surface (ERS) for Little Cayman. Highlights the level of risk to the terrestrial environment and includes the intensity, influence distance, distance decay, and overlay function. This information was combined with the flow accumulation model so that coastal outlet points could be assigned a level of risk to the marine environment due to surface run-off.

Figure 8 Simplified version of the flow accumulation model and coastal outlet points (pour points) for Little Cayman. The model highlights the major flow of water across the surface of the island, as well as the major coastal outlets, based on LIDAR elevation data.

### Cayman Brac

The terrestrial ERS risks determined for Cayman Brac were as follows:

Table 4 Terrestrial risk elements for Cayman Brac.

| Risk | Class | Intensity | Influence distance | Decay function | Overlay function |
| --- | --- | --- | --- | --- | --- |
| Landfill | N/A | 95 | 300 ft (91 m) | Linear | Maximum |
| Buildings | High | 70 | 50 ft (15 m) | Linear | Maximum |
| Airports | N/A | 70 | 300 ft (91 m) | Linear | Maximum |
| Port | N/A | 70 | 300 ft (91 m) | Linear | Maximum |
| Quarries | Dry | 60 | 300 ft (91 m) | Linear | Maximum |
| Buildings | Low | 60 | 30 ft (9 m) | Linear | Maximum |
| Roads | Primary | 50 | 50 ft (15 m) | Linear | Maximum |
| Roads | Secondary | 30 | 40 ft (12 m) | Linear | Maximum |
| Roads | Unpaved | 15 | 30 ft (9 m) | Linear | Maximum |
| Man-modified | N/A | 15 | 30 ft (9 m) | Linear | Maximum |
| Invasives | Casuarina | 5 | 10 ft (3 m) | Concave | Maximum |
| Invasives | Scaevola | 5 | 10 ft (3 m) | Concave | Maximum |

Figure 9 Terrestrial Environmental Risk Surface (ERS) for Cayman Brac. Highlights the level of risk to the terrestrial environment and includes the intensity, influence distance, distance decay, and overlay function. This information was combined with the flow accumulation model so that coastal outlet points could be assigned a level of risk to the marine environment due to surface run-off.

Figure 10 Simplified version of the flow accumulation model and coastal outlet points (pour points) for Cayman Brac. The model highlights the major flow of water across the surface of the island, as well as the major coastal outlets, based on LIDAR elevation data.

## Defining the marine Environmental Risk Surfaces

Tables 5-7 below list the identified risk elements to the marine environment for each island and their associated intensity, influence distance, decay function, and overlay function. These values were determined by DOE staff with guidance from Schill (TNC). Figures 11-13 show the modeled output for each marine ERS.

### Grand Cayman marine risks

Table 5 Marine risk elements for Grand Cayman.

| Risk | Use | Intensity | Influence distance | Decay function | Overlay function |
| --- | --- | --- | --- | --- | --- |
| Port area polygon | N/A | 90 | 2000 ft (610 m) | Concave | Maximum |
| Flow accumulation outflow points | Very high | 90 | 984 ft (300 m) | Linear | Cumulative |
| Launching ramps/docks | Very high | 70 | 500 ft (152 m) | Convex | Maximum |
| Outflows - turtle farm | N/A | 70 | 1640 ft (500 m) | Convex | Maximum |
| Outflows - CUC | N/A | 70 | 500 ft (152 m) | Convex | Cumulative |
| Marinas | N/A | 70 | 500 ft (152 m) | Convex | Maximum |
| Dredged borrow pits | N/A | 70 | 656 ft (200 m) | Concave | Maximum |
| Flow accumulation outflow points | High | 70 | 984 ft (300 m) | Linear | Cumulative |
| Hotels – rest of island | N/A | 60 | 328 ft (100 m) | Linear | Maximum |
| Spotts anchorage | N/A | 60 | 1640 ft (500 m) | Concave | Maximum |
| Canal exit points | Very high | 60 | 984 ft (300 m) | Linear | Maximum |
| Launching ramps/docks | High | 50 | 400 ft (122 m) | Convex | Maximum |
| Dolphin Cove | N/A | 50 | 800 ft (244 m) | Convex | Cumulative |
| Shallow boating navigation routes | N/A | 50 | 656 ft (200 m) | Convex | Cumulative |
| Flow accumulation outflow points | Medium | 50 | 984 ft (300 m) | Linear | Cumulative |
| Hotels - SMB | N/A | 40 | 328 ft (100 m) | Linear | Maximum |
| Canal exit points | High | 40 | 656 ft (200 m) | Linear | Maximum |
| Coastal fuel depots | N/A | 35 | 656 ft (200 m) | Linear | Maximum |
| Launching ramps/docks | Medium | 30 | 300 ft (91 m) | Convex | Maximum |
| High use boating area | N/A | 30 | 328 ft (100 m) | Linear | Maximum |
| Large ship moorings | N/A | 30 | 328 ft (100 m) | Convex | Cumulative |
| Public moorings dive pressure | Very high | 30 | 328 ft (100 m) | Convex | Cumulative |
| Flow accumulation outflow points | Low | 30 | 984 ft (300 m) | Linear | Cumulative |
| Popular snorkel sites | Very high | 25 | 100 ft (30 m) | Linear | Cumulative |
| Shore dive sites | Very high | 25 | 100 ft (30 m) | Linear | Cumulative |
| Public moorings dive pressure | High | 25 | 328 ft (100 m) | Convex | Cumulative |
| Popular snorkel sites | High | 20 | 100 ft (30 m) | Linear | Cumulative |
| Shore dive sites | High | 20 | 100 ft (30 m) | Linear | Cumulative |
| Canal exit points | Medium | 20 | 328 ft (100 m) | Linear | Maximum |
| South sound outflows | N/A | 20 | 1640 ft (500 m) | Convex | Maximum |
| Public moorings dive pressure | Medium | 20 | 328 ft (100 m) | Convex | Cumulative |
| Popular beaches | N/A | 20 | 164 ft (50 m) | Linear | Maximum |
| Watersports areas | Very high | 20 | 656 ft (200 m) | Convex | Maximum |
| Popular snorkel sites | Medium | 15 | 100 ft (30 m) | Linear | Cumulative |
| Shore dive sites | Medium | 15 | 100 ft (30 m) | Linear | Cumulative |
| Public moorings dive pressure | Low | 15 | 328 ft (100 m) | Convex | Cumulative |
| Watersports areas | High | 15 | 656 ft (200 m) | Convex | Maximum |
| Launching ramps/docks | Low | 10 | 200 ft (61 m) | Convex | Maximum |
| Popular snorkel sites | Low | 10 | 100 ft (30 m) | Linear | Cumulative |
| Shore dive sites | Low | 10 | 100 ft (30 m) | Linear | Cumulative |
| Canal exit points | Low | 10 | 33 ft (10 m) | Linear | Maximum |
| Beach access | N/A | 10 | 164 ft (50 m) | Linear | Maximum |
| Watersports areas | Medium | 10 | 656 ft (200 m) | Convex | Maximum |
| Watersports areas | Low | 5 | 656 ft (200 m) | Convex | Maximum |

Figure 11 Marine Environmental Risk Surface (ERS) for Grand Cayman. Highlights the risks to the marine environment and includes their intensity, influence distance, distance decay, and overlay function.

### Little Cayman marine risks

Table 6 Marine risk elements for Little Cayman.

| Risk | Use | Intensity | Influence distance | Decay function | Overlay function |
| --- | --- | --- | --- | --- | --- |
| Flow accumulation outflow points | Very high | 90 | 984 ft (300 m) | Linear | Cumulative |
| Salt rocks pier | N/A | 70 | 500 ft (152 m) | Convex | Maximum |
| Dredged areas | N/A | 70 | 656 ft (200 m) | Concave | Maximum |
| Flow accumulation outflow points | High | 70 | 984 ft (300 m) | Linear | Cumulative |
| Shallow boating navigation routes | N/A | 50 | 656 ft (200 m) | Convex | Cumulative |
| Flow accumulation outflow points | Medium | 50 | 984 ft (300 m) | Linear | Cumulative |
| Hotels | N/A | 40 | 328 ft (100 m) | Linear | Maximum |
| Launching ramps/docks | Medium | 30 | 300 ft (91 m) | Convex | Maximum |
| High use boating area | N/A | 30 | 328 ft (100 m) | Linear | Maximum |
| Public moorings dive pressure | Very high | 30 | 328 ft (100 m) | Convex | Cumulative |
| Flow accumulation outflow points | Low | 30 | 984 ft (300 m) | Linear | Cumulative |
| Public moorings dive pressure | High | 25 | 328 ft (100 m) | Convex | Cumulative |
| Docks | N/A | 25 | 33 ft (10 m) | Linear | Maximum |
| Public moorings dive pressure | Medium | 20 | 328 ft (100 m) | Convex | Cumulative |
| Popular beaches | N/A | 20 | 164 ft (50 m) | Linear | Maximum |
| Popular snorkel sites | Medium | 15 | 100 ft (30 m) | Linear | Cumulative |
| Shore dive sites | Medium | 15 | 100 ft (30 m) | Linear | Maximum |
| Public moorings dive pressure | Low | 15 | 328 ft (100 m) | Convex | Cumulative |
| Launching ramps/docks | Low | 10 | 200 ft (61 m) | Convex | Maximum |

Figure 12 Marine Environmental Risk Surface (ERS) for Little Cayman. Highlights the risks to the marine environment and includes their intensity, influence distance, distance decay, and overlay function.

### Cayman Brac marine risks

Table 7 Marine risk elements for Cayman Brac.

| Risk | Use | Intensity | Influence distance | Decay function | Overlay function |
| --- | --- | --- | --- | --- | --- |
| Flow accumulation outflow points | Very high | 90 | 984 ft (300 m) | Linear | Cumulative |
| Port pier | N/A | 70 | 500 ft (152 m) | Convex | Maximum |
| Dredged areas | N/A | 70 | 656 ft (200 m) | Concave | Maximum |
| Flow accumulation outflow points | High | 70 | 984 ft (300 m) | Linear | Cumulative |
| Launching ramps/docks | High | 50 | 400 ft (122 m) | Linear | Maximum |
| Flow accumulation outflow points | Medium | 50 | 984 ft (300 m) | Linear | Cumulative |
| Hotels | N/A | 40 | 328 ft (100 m) | Linear | Maximum |
| Launch ramps | Medium | 30 | 300 ft (91 m) | Linear | Maximum |
| High use boating area | N/A | 30 | 328 ft (100 m) | Linear | Maximum |
| Flow accumulation outflow points | Low | 30 | 984 ft (300 m) | Linear | Cumulative |
| Public moorings dive pressure | High | 25 | 328 ft (100 m) | Convex | Cumulative |
| Docks | N/A | 25 | 33 ft (10 m) | Linear | Maximum |
| Outflows | N/A | 20 | 1640 ft (500 m) | Convex | Maximum |
| Public moorings dive pressure | Medium | 20 | 328 ft (100 m) | Convex | Cumulative |
| Popular beaches | N/A | 20 | 164 ft (50 m) | Linear | Maximum |
| Shore dive sites | Medium | 15 | 100 ft (30 m) | Linear | Maximum |
| Public moorings dive pressure | Low | 15 | 328 ft (100 m) | Convex | Cumulative |
| Launching ramps/docks | Low | 10 | 200 ft (61 m) | Linear | Maximum |

Figure 13 Marine Environmental Risk Surface (ERS) for Cayman Brac. Highlights the risks to the marine environment and includes their intensity, influence distance, distance decay, and overlay function.

# Conservation targets definition and goal setting

To compile a list of conservation targets for Grand Cayman, all marine habitat features were considered that had been recently mapped with high precision using high-resolution color aerial photography (12 cm resolution) and satellite imagery (Quickbird 60 cm pan-sharpened) (Cottam et al, 2009). Further, existing information on critical habitats and important species were also considered. Broad categories were created to group these target candidates into the general classes of: 1) Outside Lagoon Benthic 2) Lagoon Benthic, 3) Shoreline 4) Wetland 5) Protected Land 6) Proposed Land For Protection and 7) Species. In order for a conservation target to be included, it was required to be a have a spatial extent that was complete for all of the Cayman Islands. For example, while it is understood where some important species like Elkhorn coral (Acropora palmata) and Staghorn coral (A. cervicornis) generally exist, they were not included in the target list as they have not been comprehensively mapped and could spatially skew Marxan results by steering outputs towards or away from areas that do not represent a complete distribution of the species.

The final list of conservation targets (Table 8) indicate the corresponding conservation goals that were assigned by DOE staff. Base goals are the percentage of each conservation target that Marxan tries to include in the ‘Base Goals’ scenario solution. Targets that were considered most important were given a value of 40% of their total area and less important targets were assigned 30% of their total area. Targets of high importance were determined by their ability to support a high level of biodiversity as well as their importance to key species. Additional goal scenarios used in Marxan include the protection of 40% and 30% of each target respectively within each strata. The ‘Penalty Factor’ determines the priority with which the algorithm meets an individual target’s representation goal. In other words, it is a cost imposed for failing to meet the goals and is primarily based on the importance of meeting the target’s goal. For the purpose of this project, Penalty Factor values were set to a constant value of 1 for all features. This was done because we considered all conservation targets of equal importance for meeting the goals. High goals were also included in the table in order to generate an ‘ultra-conservative’ scenario where Marxan was tasked with trying to protect 80% of the representation of the most important targets. A feature ID is a unique value assigned that Marxan uses to identify each conservation target. The feature ID was combined with the strata ID (above) to provide a unique identifier for all combinations of strata and targets (Appendix 1). Note that not all conservation targets are found on all three islands.

Table 8 Marine conservation targets for all three Cayman Islands

| Group | Conservation target | Feature ID | Base goals | Penalty factor | High goals |
| --- | --- | --- | --- | --- | --- |
| Outside lagoon benthic | Aggregate reef | 101 | 40 | High | 80 |
| Outside lagoon benthic | Aggregated patch reef | 102 | 40 | High | 80 |
| Outside lagoon benthic | Beach rock | 103 | 30 | Medium |  |
| Outside lagoon benthic | Colonized hardbottom | 104 | 40 | Medium |  |
| Outside lagoon benthic | Individual patch reef | 105 | 40 | Medium | 80 |
| Outside lagoon benthic | Reef crest | 106 | 40 | High | 80 |
| Outside lagoon benthic | Rubble | 107 | 30 | Low |  |
| Outside lagoon benthic | Sand | 108 | 30 | High |  |
| Outside lagoon benthic | Spur and groove | 109 | 40 | High | 80 |
| Outside lagoon benthic | Uncolonized hardbottom | 110 | 30 | Low |  |
| Lagoon benthic | Backreef | 111 | 40 | High | 80 |
| Lagoon benthic | Beach rock | 112 | 30 | Medium |  |
| Lagoon benthic | Hardbottom | 113 | 30 | Medium |  |
| Lagoon benthic | Lagoonal coral | 114 | 40 | High | 80 |
| Lagoon benthic | Mud | 115 | 30 | Medium |  |
| Lagoon benthic | Seagrass beds | 116 | 40 | High | 80 |
| Lagoon benthic | Sediment | 117 | 30 | High |  |
| Lagoon benthic | Vegetated sand | 118 | 40 | High |  |
| Shoreline | Cliff | 119 | 40 | High |  |
| Shoreline | Cobble | 120 | 30 | Medium |  |
| Shoreline | Cobble mosaic | 121 | 30 | Medium |  |
| Shoreline | Ironshore | 122 | 30 | Medium |  |
| Shoreline | Ironshore mosaic | 123 | 30 | Medium |  |
| Shoreline | Rock | 124 | 30 | Low |  |
| Shoreline | Rock mosaic | 125 | 30 | Low |  |
| Shoreline | Sand | 126 | 30 | Medium |  |
| Shoreline | Sand mosaic | 127 | 30 | Medium |  |
| Wetland | Mangrove | 128 | 40 | High | 80 |
| Protected land | Crown land | 129 | 40 | High |  |
| Species | Main turtle nesting beaches | 139 | 40 | High | 80 |

# Design of planning units

Planning units are defined by overlaying the planning region with a grid of squares or lattice of hexagons (Figure 14). These polygon features contain all the necessary information for Marxan to run and allow comparison and selection between candidate areas. They must capture all the areas that can possibly be selected as part of the reserve system and their size should be at a scale appropriate for both the ecological features that are wanted to be captured and the size of the protected areas likely to be implemented (Game and Grantham, 2008).

After review of the high degree of accuracy and large amount of data that is available, as well as the relatively small ‘shelf’ area surrounding the Cayman Islands, it was determined that a 1 acre (0.405 ha) planning unit would be optimum to capture small spatial differences within the input datasets. There were 90,391 one-acre planning units for Grand Cayman, 14,254 for Little Cayman, and 15,477 for Cayman Brac. A hexagon shape grid was used since it provides several advantages compared to other methods such as squared or circular plots. Most important in the context of evaluating patterns is the equidistant nature of the grid (Figure 14). This results in a more balanced modeling environment and facilitates data analysis and circumvents statistical and logical problems (compared to squared or circular plots) (Jurasinski and Beierkuhnlein, 2006).

Figure 14 Conservation features are overlaid on top of risks and each planning unit (1 acre) is assigned a score based on the level of cost associated with obtaining a conservation feature within each unit. Conservation feature layer has been clipped for display purposes but should match the extents of the risk surface and planning units.

# Inclusion of status layers

The use of ‘status layers’ is an important aspect of Marxan modeling. These layers are required if a user plans to run a scenario where the planning units that overlap declared protected areas or special interest areas are to be “locked-in” or fixed in the reserve, thus always included in the solution. These planning units are first considered when Marxan attempts to meet the conservation goals. If goals are met within these areas, no additional planning units are selected for that particular conservation target (Schill and Raber, 2008).

To this end, existing no-take protected areas were initially locked-in as a ‘Strict’ status layer (Environmental Zones, Marine Parks, and Wildlife Interaction Zones; Figure 15) for preliminary Marxan runs. For subsequent models, it was deemed beneficial to also include all protected areas in a set of ‘All’ status layers. A third option was added using No status layers– that is all areas have an equal chance of being included in the modeled scenarios. The ‘No status layers’ option was included so that the output Marxan solution was not biased towards an area simply because it is already protected, and therefore provides an estimate of what areas should be protected if no marine protected areas pre-existed.

Figure 15‘No-Take Protected Areas’ Strict status layers for all Cayman Islands – planning units within these areas are first considered when attempting to meet conservation goals. ‘All Protected Areas’ and ‘No Protected Areas’ status layers were also considered in the final Marxan output scenarios.

# Marxan results

In order to run Marxan, the software requires the creation of several input text files. These files are based on the planning units, strata, targets, risk surface, goals, status layers and other important parameters that Marxan needs to run. For this exercise, we used the Protected Area Tools (PAT) for ArcGIS 9.3 (http://gg.usm.edu/pat/) to create the input files using the combined strata and conservation targets that were previously created in ESRI spatial data format (shapefile and/or feature class) (Schill and Raber, 2011). The PAT software has the ability to create the ERS model, create hexagons for planning units, prepare targets, and extract the abundance of each conservation target within each planning unit as well as summarize the associated risks. These input files can then be used in Zonae Cogito (ZC) decision support software that facilitates Marxan runs and visualization of model outputs (Watts et al, 2009).

One parameter of particular importance when creating Marxan solutions is the Boundary Length Modifier (BLM). The BLM determines the relative importance placed on minimizing the perimeter relative to minimizing area. This is a weighting factor that can be used to control the spatial aggregation (clumping and/or fragmentation) of selected assessment units. BLM is an important factor as it determines whether the output contains a high number of small sites (low BLM), or fewer, larger sites (high BLM). Spatial optimization is achieved by minimizing both area and length of perimeter between assessment units; therefore testing the influence of this parameter is essential. Upon calibration, it was determined that the optimal BLM was a value between 75 – 150. Figure 16 shows the final versions of a modeled Marxan scenario for the marine environment in Grand Cayman, Little Cayman, and Cayman Brac each with a BLM of 150. A text version of a mapped solution showing target amounts of conservation features and the actual amount held in the solution may be found in Appendix 2.

Figure 16 Modeled Marxan scenarios for all Cayman Islands’ marine environment. The maps display a scenario where the ERS is included, ‘No-Take Marine Parks’ Strict status layers are locked in, 30% and 40% (Base) protection goals are in place, a BLM of 150, and a constant SPF value of 1 for all conservation targets used.

# Discussion

Marxan provides decision support for conservation planning, however it is just one tool among many when determining how best to design or expand a marine protected area system. In addition to Marxan solutions, other Decision Support Tools (DSTs) were utilized to assist in determining the best marine park design. Additional DSTs used for determining the final MPA design were as follows:

- Reef Health Index (Simplified Integrated Reef Health Index) data at 63 permanent sites around the Cayman Islands (www.healthyreefs.org). 55 sites had all variables included in the index, and 8 sites had only coral-related data included in the index

- Fishing pressure location and intensity

- Existing Marine Park system location

- Local knowledge of current usage and issues regarding enforcement of current marine park laws

- Locations of public moorings

- Existing parcel boundaries and/or landmarks for ease of signage

- Local knowledge of locations of important and endangered species such as Elkhorn coral (Acropora palmata) and Staghorn coral (Acropora cervicornis)

- Results of previous local studies on specific species (i.e. conch, sharks, turtles – nesting and in-water, etc.)

- Locally determined areas of potential resistance from user groups

## Reef Health Index

A relative measure of reef health was considered in order to identify those areas of reef which might be more resilient than others and therefore of higher conservation value. An index of reef health was calculated using the most recently collected data (2011) based on the Simplified Integrated Reef Health Index designed by the Healthy Reefs Initiative (www.healthyreefs.org) specifically for measuring the health of the Mesoamerican Reef Ecosystem (Figure 17). The index was created using the experience, data, and perspectives of its scientific review committee, and incorporates measures of live coral cover, fleshy macro-algal cover, herbivorous fish abundance and commercial fish abundance (Appendix 3). The more comprehensive Healthy Reefs Integrated Reef Health Index (IRHI) which calculates a measure of reef health based on additional indicators, such as coral disease and recruitment, Diadema abundance and algal heights (needed for a Macro-algal index), could not be applied since datasets for all seven of the IRHI indicators were not available. Reef health index data was only available for certain field sites and did not cover the entire project extent (Figure 18) and as such its incorporation into Marxan was not considered a possibility. Incorporation was rejected on the grounds that methods of inclusion (e.g. interpolation of data between sites, assigning large areas of shelf coral reef using a single site’s value, etc.) would introduce error and disrupt the confidence of the data (due to high localized variance in the marine environment), and limiting the values to isolated sites would bias Marxan outputs. As such, reef health index data was used as a DST to guide allocation of protected area boundaries alongside modeled Marxan solutions.

Figure 17 Threshold values by indicator for the Simplified Integrated Reef Health Index (SIHRI). Source: Healthy Reefs Initiative (2010) Report Card for the Mesoamerican Reef.

Figure 18 Reef health index (Simplified Integrated Reef Health Index), combining coral cover, fleshy macro-algal cover, herbivorous fish abundance and commercial fish abundance at 63 permanent sampling sites; www.healthyreefs.org. (Regional threshold values: 1=Critical, 2=Poor, 3=Fair, 4=Good, 5=Very good).

## Fishing pressure location and intensity

Data incorporated as a DST into the protected area planning process on the location and intensity of local fishing pressure was acquired from extensive 2011survey results (Figure 19). Data included detailed information on catch size, target species, and the distribution and effort of fishing monthly fishing activities. The views of fishers on the marine environment, and designated marine protected areas around the islands, were also gauged through a series of multiple-choice questions.

The data could not be incorporated into Marxan since it was collected at too coarse a scale and lacked the high degree of spatial confidence associated with applied Marxan data. It was also determined that whilst an understanding of the location of fishing effort is critical to the risks to the marine environment, fishing effort will likely be mobile according to location of fish which can be determined by condition of reefs. The data therefore served as an additional DST, providing valuable guidance on the selection of important marine protected areas according to usage and potential user conflicts.

Figure 19 Spatial distribution of fishing effort over a monthly period on all Cayman Islands. Star symbols show locations of the major boat launch areas.

## Next Steps

Guided by generated Marxan solutions and complementary DSTs, the DOE and partners drafted recommended enhancements to the system of marine protected areas in the Cayman Islands for immediate approval by Cabinet. These recommendations incorporate careful analysis of generated Marxan solutions, complementary DSTs, and extensive public feedback so to create an optimum conservation solution for all user groups now and into the future within the Cayman Islands.

# References

Ball, I.R., H.P. Possingham, and M. Watts. 2009. Marxan and relatives: Software for spatial conservation prioritization. Chapter 14: Pages 185-195 in Spatial conservation prioritization: Quantitative methods and computational tools. Eds Moilanen, A., K.A. Wilson, and H.P. Possingham. Oxford University Press, Oxford, UK.

Schill, S. and G. Raber. 2011. Protected Area Tools (PAT) for ArcGIS 10 (version 4.0) software http://www.gispatools.org. The Nature Conservancy, Arlington, Virginia.

Schill, S. and G. Raber. 2011. Protected Area Tools (PAT) for ArcGIS 9.3 (version 3.0) software

http://www.gg.usm.edu/pat/. The Nature Conservancy, Arlington, Virginia.

Watts, M.E, I.R. Ball, R.R. Stewart, C.J. Klein, K. Wilson, C. Steinback, R. Lourival, L. Kircher, H.P. Possingham. 2009. Marxan with Zones: software for optimal conservation based land- and sea-use zoning, Environmental Modeling & Software, doi:10.1016/j.envsoft. 2009.06.005.

Cottam, M., Olynik, J., Blumenthal, J., Godbeer, K.D., Gibb, J., Bothwell, J., Burton, F.J., Bradley, P.E., Band, A., Austin, T., Bush, P., Johnson, B.J., Hurlston, L., Bishop, L., McCoy, C., Parsons, G., Kirkconnell, J., Halford, S. and Ebanks-Petrie, G. 2009. Cayman Islands National Biodiversity Action Plan. Cayman Islands Government. Department of Environment.

Game, E. T. and H. S. Grantham. 2008. Marxan User Manual: For Marxan version 1.8.10. University of Queensland, St. Lucia, Queensland, Australia, and Pacific Marine Analysis and Research Association, Vancouver, British Columbia, Canada.

Jurasinski, G. and C. Beierkuhnlein. 2006. Spatial Patterns of Biodiversity-Assessing Vegetation Using Hexagonal Grids. Biology and Environment, Vol 106B, No. 3, 401-41.

Appendix 1 Final list of Conservation Targets.

| Island | Conservation target label | Strata ID | Feature ID | Target ID |
| --- | --- | --- | --- | --- |
| Grand Cayman | North Sound Outside Lagoon Benthic Aggregate Reef | 10 | 101 | 10101 |
|  | North Sound Outside Lagoon Benthic Aggregated Patch Reef | 10 | 102 | 10102 |
|  | North Sound Outside Lagoon Benthic Beach Rock | 10 | 103 | 10103 |
|  | North Sound Outside Lagoon Benthic Colonized Hardbottom | 10 | 104 | 10104 |
|  | North Sound Outside Lagoon Benthic Individual Patch Reef | 10 | 105 | 10105 |
|  | North Sound Outside Lagoon Benthic Reef Crest | 10 | 106 | 10106 |
|  | North Sound Outside Lagoon Benthic Rubble | 10 | 107 | 10107 |
|  | North Sound Outside Lagoon Benthic Sand | 10 | 108 | 10108 |
|  | North Sound Outside Lagoon Benthic Spur and Groove | 10 | 109 | 10109 |
|  | North Sound Outside Lagoon Benthic Uncolonized Hardbottom | 10 | 110 | 10110 |
|  | North Sound Lagoon Benthic Backreef | 10 | 111 | 10111 |
|  | North Sound Lagoon Benthic Beach Rock | 10 | 112 | 10112 |
|  | North Sound Lagoon Benthic Hardbottom | 10 | 113 | 10113 |
|  | North Sound Lagoon Benthic Lagoonal Coral | 10 | 114 | 10114 |
|  | North Sound Lagoon Benthic Mud | 10 | 115 | 10115 |
|  | North Sound Lagoon Benthic Seagrass Beds | 10 | 116 | 10116 |
|  | North Sound Lagoon Benthic Sediment | 10 | 117 | 10117 |
|  | North Sound Lagoon Benthic Vegetated Sand | 10 | 118 | 10118 |
|  | North Sound Shoreline Cliff | 10 | 119 | 10119 |
|  | North Sound Shoreline Cobble | 10 | 120 | 10120 |
|  | North Sound Shoreline Cobble Mosaic | 10 | 121 | 10121 |
|  | North Sound Shoreline Ironshore | 10 | 122 | 10122 |
|  | North Sound Shoreline Ironshore Mosaic | 10 | 123 | 10123 |
|  | North Sound Shoreline Rock | 10 | 124 | 10124 |
|  | North Sound Shoreline Rock Mosaic | 10 | 125 | 10125 |
|  | North Sound Shoreline Sand | 10 | 126 | 10126 |
|  | North Sound Shoreline Sand Mosaic | 10 | 127 | 10127 |
|  | North Sound Wetland Mangrove | 10 | 128 | 10128 |
|  | North Sound Protected Land Crown Land | 10 | 129 | 10129 |
|  | North Sound Species Main Turtle Nesting Beaches | 10 | 139 | 10139 |
|  | West Side Outside Lagoon Benthic Aggregate Reef | 20 | 101 | 20101 |
|  | West Side Outside Lagoon Benthic Aggregated Patch Reef | 20 | 102 | 20102 |
|  | West Side Outside Lagoon Benthic Beach Rock | 20 | 103 | 20103 |
|  | West Side Outside Lagoon Benthic Colonized Hardbottom | 20 | 104 | 20104 |
|  | West Side Outside Lagoon Benthic Individual Patch Reef | 20 | 105 | 20105 |
|  | West Side Outside Lagoon Benthic Reef Crest | 20 | 106 | 20106 |
|  | West Side Outside Lagoon Benthic Rubble | 20 | 107 | 20107 |
|  | West Side Outside Lagoon Benthic Sand | 20 | 108 | 20108 |
|  | West Side Outside Lagoon Benthic Spur and Groove | 20 | 109 | 20109 |
|  | West Side Outside Lagoon Benthic Uncolonized Hardbottom | 20 | 110 | 20110 |
|  | West Side Lagoon Benthic Backreef | 20 | 111 | 20111 |
|  | West Side Lagoon Benthic Beach Rock | 20 | 112 | 20112 |
|  | West Side Lagoon Benthic Hardbottom | 20 | 113 | 20113 |
|  | West Side Lagoon Benthic Lagoonal Coral | 20 | 114 | 20114 |
|  | West Side Lagoon Benthic Mud | 20 | 115 | 20115 |
|  | West Side Lagoon Benthic Seagrass Beds | 20 | 116 | 20116 |
|  | West Side Lagoon Benthic Sediment | 20 | 117 | 20117 |
|  | West Side Lagoon Benthic Vegetated Sand | 20 | 118 | 20118 |
|  | West Side Shoreline Cliff | 20 | 119 | 20119 |
|  | West Side Shoreline Cobble | 20 | 120 | 20120 |
|  | West Side Shoreline Cobble Mosaic | 20 | 121 | 20121 |
|  | West Side Shoreline Ironshore | 20 | 122 | 20122 |
|  | West Side Shoreline Ironshore Mosaic | 20 | 123 | 20123 |
|  | West Side Shoreline Rock | 20 | 124 | 20124 |
|  | West Side Shoreline Rock Mosaic | 20 | 125 | 20125 |
|  | West Side Shoreline Sand | 20 | 126 | 20126 |
|  | West Side Shoreline Sand Mosaic | 20 | 127 | 20127 |
|  | West Side Wetland Mangrove | 20 | 128 | 20128 |
|  | West Side Protected Land Crown Land | 20 | 129 | 20129 |
|  | West Side Species Main Turtle Nesting Beaches | 20 | 139 | 20139 |
|  | South Side Outside Lagoon Benthic Aggregate Reef | 30 | 101 | 30101 |
|  | South Side Outside Lagoon Benthic Aggregated Patch Reef | 30 | 102 | 30102 |
|  | South Side Outside Lagoon Benthic Beach Rock | 30 | 103 | 30103 |
|  | South Side Outside Lagoon Benthic Colonized Hardbottom | 30 | 104 | 30104 |
|  | South Side Outside Lagoon Benthic Individual Patch Reef | 30 | 105 | 30105 |
|  | South Side Outside Lagoon Benthic Reef Crest | 30 | 106 | 30106 |
|  | South Side Outside Lagoon Benthic Rubble | 30 | 107 | 30107 |
|  | South Side Outside Lagoon Benthic Sand | 30 | 108 | 30108 |
|  | South Side Outside Lagoon Benthic Spur and Groove | 30 | 109 | 30109 |
|  | South Side Outside Lagoon Benthic Uncolonized Hardbottom | 30 | 110 | 30110 |
|  | South Side Lagoon Benthic Backreef | 30 | 111 | 30111 |
|  | South Side Lagoon Benthic Beach Rock | 30 | 112 | 30112 |
|  | South Side Lagoon Benthic Hardbottom | 30 | 113 | 30113 |
|  | South Side Lagoon Benthic Lagoonal Coral | 30 | 114 | 30114 |
|  | South Side Lagoon Benthic Mud | 30 | 115 | 30115 |
|  | South Side Lagoon Benthic Seagrass Beds | 30 | 116 | 30116 |
|  | South Side Lagoon Benthic Sediment | 30 | 117 | 30117 |
|  | South Side Lagoon Benthic Vegetated Sand | 30 | 118 | 30118 |
|  | South Side Shoreline Cliff | 30 | 119 | 30119 |
|  | South Side Shoreline Cobble | 30 | 120 | 30120 |
|  | South Side Shoreline Cobble Mosaic | 30 | 121 | 30121 |
|  | South Side Shoreline Ironshore | 30 | 122 | 30122 |
|  | South Side Shoreline Ironshore Mosaic | 30 | 123 | 30123 |
|  | South Side Shoreline Rock | 30 | 124 | 30124 |
|  | South Side Shoreline Rock Mosaic | 30 | 125 | 30125 |
|  | South Side Shoreline Sand | 30 | 126 | 30126 |
|  | South Side Shoreline Sand Mosaic | 30 | 127 | 30127 |
|  | South Side Wetland Mangrove | 30 | 128 | 30128 |
|  | South Side Protected Land Crown Land | 30 | 129 | 30129 |
|  | South Side Species Main Turtle Nesting Beaches | 30 | 139 | 30139 |
|  | North Side Outside Lagoon Benthic Aggregate Reef | 40 | 101 | 40101 |
|  | North Side Outside Lagoon Benthic Aggregated Patch Reef | 40 | 102 | 40102 |
|  | North Side Outside Lagoon Benthic Beach Rock | 40 | 103 | 40103 |
|  | North Side Outside Lagoon Benthic Colonized Hardbottom | 40 | 104 | 40104 |
|  | North Side Outside Lagoon Benthic Individual Patch Reef | 40 | 105 | 40105 |
|  | North Side Outside Lagoon Benthic Reef Crest | 40 | 106 | 40106 |
|  | North Side Outside Lagoon Benthic Rubble | 40 | 107 | 40107 |
|  | North Side Outside Lagoon Benthic Sand | 40 | 108 | 40108 |
|  | North Side Outside Lagoon Benthic Spur and Groove | 40 | 109 | 40109 |
|  | North Side Outside Lagoon Benthic Uncolonized Hardbottom | 40 | 110 | 40110 |
|  | North Side Lagoon Benthic Backreef | 40 | 111 | 40111 |
|  | North Side Lagoon Benthic Beach Rock | 40 | 112 | 40112 |
|  | North Side Lagoon Benthic Hardbottom | 40 | 113 | 40113 |
|  | North Side Lagoon Benthic Lagoonal Coral | 40 | 114 | 40114 |
|  | North Side Lagoon Benthic Mud | 40 | 115 | 40115 |
|  | North Side Lagoon Benthic Seagrass Beds | 40 | 116 | 40116 |
|  | North Side Lagoon Benthic Sediment | 40 | 117 | 40117 |
|  | North Side Lagoon Benthic Vegetated Sand | 40 | 118 | 40118 |
|  | North Side Shoreline Cliff | 40 | 119 | 40119 |
|  | North Side Shoreline Cobble | 40 | 120 | 40120 |
|  | North Side Shoreline Cobble Mosaic | 40 | 121 | 40121 |
|  | North Side Shoreline Ironshore | 40 | 122 | 40122 |
|  | North Side Shoreline Ironshore Mosaic | 40 | 123 | 40123 |
|  | North Side Shoreline Rock | 40 | 124 | 40124 |
|  | North Side Shoreline Rock Mosaic | 40 | 125 | 40125 |
|  | North Side Shoreline Sand | 40 | 126 | 40126 |
|  | North Side Shoreline Sand Mosaic | 40 | 127 | 40127 |
|  | North Side Wetland Mangrove | 40 | 128 | 40128 |
|  | North Side Protected Land Crown Land | 40 | 129 | 40129 |
|  | North Side Species Main Turtle Nesting Beaches | 40 | 139 | 40139 |
|  | East End Outside Lagoon Benthic Aggregate Reef | 50 | 101 | 50101 |
|  | East End Outside Lagoon Benthic Aggregated Patch Reef | 50 | 102 | 50102 |
|  | East End Outside Lagoon Benthic Beach Rock | 50 | 103 | 50103 |
|  | East End Outside Lagoon Benthic Colonized Hardbottom | 50 | 104 | 50104 |
|  | East End Outside Lagoon Benthic Individual Patch Reef | 50 | 105 | 50105 |
|  | East End Outside Lagoon Benthic Reef Crest | 50 | 106 | 50106 |
|  | East End Outside Lagoon Benthic Rubble | 50 | 107 | 50107 |
|  | East End Outside Lagoon Benthic Sand | 50 | 108 | 50108 |
|  | East End Outside Lagoon Benthic Spur and Groove | 50 | 109 | 50109 |
|  | East End Outside Lagoon Benthic Uncolonized Hardbottom | 50 | 110 | 50110 |
|  | East End Lagoon Benthic Backreef | 50 | 111 | 50111 |
|  | East End Lagoon Benthic Beach Rock | 50 | 112 | 50112 |
|  | East End Lagoon Benthic Hardbottom | 50 | 113 | 50113 |
|  | East End Lagoon Benthic Lagoonal Coral | 50 | 114 | 50114 |
|  | East End Lagoon Benthic Mud | 50 | 115 | 50115 |
|  | East End Lagoon Benthic Seagrass Beds | 50 | 116 | 50116 |
|  | East End Lagoon Benthic Sediment | 50 | 117 | 50117 |
|  | East End Lagoon Benthic Vegetated Sand | 50 | 118 | 50118 |
|  | East End Shoreline Cliff | 50 | 119 | 50119 |
|  | East End Shoreline Cobble | 50 | 120 | 50120 |
|  | East End Shoreline Cobble Mosaic | 50 | 121 | 50121 |
|  | East End Shoreline Ironshore | 50 | 122 | 50122 |
|  | East End Shoreline Ironshore Mosaic | 50 | 123 | 50123 |
|  | East End Shoreline Rock | 50 | 124 | 50124 |
|  | East End Shoreline Rock Mosaic | 50 | 125 | 50125 |
|  | East End Shoreline Sand | 50 | 126 | 50126 |
|  | East End Shoreline Sand Mosaic | 50 | 127 | 50127 |
|  | East End Wetland Mangrove | 50 | 128 | 50128 |
|  | East End Protected Land Crown Land | 50 | 129 | 50129 |
|  | East End Species Main Turtle Nesting Beaches | 50 | 139 | 50139 |
| Little Cayman | Little Cayman North Outside Lagoon Benthic Aggregate Reef | 60 | 101 | 60101 |
|  | Little Cayman North Outside Lagoon Benthic Aggregated Patch Reef | 60 | 102 | 60102 |
|  | Little Cayman North Outside Lagoon Benthic Beach Rock | 60 | 103 | 60103 |
|  | Little Cayman North Outside Lagoon Benthic Colonized Hardbottom | 60 | 104 | 60104 |
|  | Little Cayman North Outside Lagoon Benthic Individual Patch Reef | 60 | 105 | 60105 |
|  | Little Cayman North Outside Lagoon Benthic Reef Crest | 60 | 106 | 60106 |
|  | Little Cayman North Outside Lagoon Benthic Rubble | 60 | 107 | 60107 |
|  | Little Cayman North Outside Lagoon Benthic Sand | 60 | 108 | 60108 |
|  | Little Cayman North Outside Lagoon Benthic Spur and Groove | 60 | 109 | 60109 |
|  | Little Cayman North Outside Lagoon Benthic Uncolonized Hardbottom | 60 | 110 | 60110 |
|  | Little Cayman North Lagoon Benthic Backreef | 60 | 111 | 60111 |
|  | Little Cayman North Lagoon Benthic Beach Rock | 60 | 112 | 60112 |
|  | Little Cayman North Lagoon Benthic Hardbottom | 60 | 113 | 60113 |
|  | Little Cayman North Lagoon Benthic Lagoonal Coral | 60 | 114 | 60114 |
|  | Little Cayman North Lagoon Benthic Mud | 60 | 115 | 60115 |
|  | Little Cayman North Lagoon Benthic Seagrass Beds | 60 | 116 | 60116 |
|  | Little Cayman North Lagoon Benthic Sediment | 60 | 117 | 60117 |
|  | Little Cayman North Lagoon Benthic Vegetated Sand | 60 | 118 | 60118 |
|  | Little Cayman North Shoreline Cliff | 60 | 119 | 60119 |
|  | Little Cayman North Shoreline Cobble | 60 | 120 | 60120 |
|  | Little Cayman North Shoreline Cobble Mosaic | 60 | 121 | 60121 |
|  | Little Cayman North Shoreline Ironshore | 60 | 122 | 60122 |
|  | Little Cayman North Shoreline Ironshore Mosaic | 60 | 123 | 60123 |
|  | Little Cayman North Shoreline Rock | 60 | 124 | 60124 |
|  | Little Cayman North Shoreline Rock Mosaic | 60 | 125 | 60125 |
|  | Little Cayman North Shoreline Sand | 60 | 126 | 60126 |
|  | Little Cayman North Shoreline Sand Mosaic | 60 | 127 | 60127 |
|  | Little Cayman North Wetland Mangrove | 60 | 128 | 60128 |
|  | Little Cayman North Protected Land Crown Land | 60 | 129 | 60129 |
|  | Little Cayman North Species Main Turtle Nesting Beaches | 60 | 139 | 60139 |
|  | Little Cayman South Outside Lagoon Benthic Aggregate Reef | 70 | 101 | 70101 |
|  | Little Cayman South Outside Lagoon Benthic Aggregated Patch Reef | 70 | 102 | 70102 |
|  | Little Cayman South Outside Lagoon Benthic Beach Rock | 70 | 103 | 70103 |
|  | Little Cayman South Outside Lagoon Benthic Colonized Hardbottom | 70 | 104 | 70104 |
|  | Little Cayman South Outside Lagoon Benthic Individual Patch Reef | 70 | 105 | 70105 |
|  | Little Cayman South Outside Lagoon Benthic Reef Crest | 70 | 106 | 70106 |
|  | Little Cayman South Outside Lagoon Benthic Rubble | 70 | 107 | 70107 |
|  | Little Cayman South Outside Lagoon Benthic Sand | 70 | 108 | 70108 |
|  | Little Cayman South Outside Lagoon Benthic Spur and Groove | 70 | 109 | 70109 |
|  | Little Cayman South Outside Lagoon Benthic Uncolonized Hardbottom | 70 | 110 | 70110 |
|  | Little Cayman South Lagoon Benthic Backreef | 70 | 111 | 70111 |
|  | Little Cayman South Lagoon Benthic Beach Rock | 70 | 112 | 70112 |
|  | Little Cayman South Lagoon Benthic Hardbottom | 70 | 113 | 70113 |
|  | Little Cayman South Lagoon Benthic Lagoonal Coral | 70 | 114 | 70114 |
|  | Little Cayman South Lagoon Benthic Mud | 70 | 115 | 70115 |
|  | Little Cayman South Lagoon Benthic Seagrass Beds | 70 | 116 | 70116 |
|  | Little Cayman South Lagoon Benthic Sediment | 70 | 117 | 70117 |
|  | Little Cayman South Lagoon Benthic Vegetated Sand | 70 | 118 | 70118 |
|  | Little Cayman South Shoreline Cliff | 70 | 119 | 70119 |
|  | Little Cayman South Shoreline Cobble | 70 | 120 | 70120 |
|  | Little Cayman South Shoreline Cobble Mosaic | 70 | 121 | 70121 |
|  | Little Cayman South Shoreline Ironshore | 70 | 122 | 70122 |
|  | Little Cayman South Shoreline Ironshore Mosaic | 70 | 123 | 70123 |
|  | Little Cayman South Shoreline Rock | 70 | 124 | 70124 |
|  | Little Cayman South Shoreline Rock Mosaic | 70 | 125 | 70125 |
|  | Little Cayman South Shoreline Sand | 70 | 126 | 70126 |
|  | Little Cayman South Shoreline Sand Mosaic | 70 | 127 | 70127 |
|  | Little Cayman South Wetland Mangrove | 70 | 128 | 70128 |
|  | Little Cayman South Protected Land Crown Land | 70 | 129 | 70129 |
|  | Little Cayman South Species Main Turtle Nesting Beaches | 70 | 139 | 70139 |
| Cayman Brac | Cayman Brac North Outside Lagoon Benthic Aggregate Reef | 80 | 101 | 80101 |
|  | Cayman Brac North Outside Lagoon Benthic Aggregated Patch Reef | 80 | 102 | 80102 |
|  | Cayman Brac North Outside Lagoon Benthic Beach Rock | 80 | 103 | 80103 |
|  | Cayman Brac North Outside Lagoon Benthic Colonized Hardbottom | 80 | 104 | 80104 |
|  | Cayman Brac North Outside Lagoon Benthic Individual Patch Reef | 80 | 105 | 80105 |
|  | Cayman Brac North Outside Lagoon Benthic Reef Crest | 80 | 106 | 80106 |
|  | Cayman Brac North Outside Lagoon Benthic Rubble | 80 | 107 | 80107 |
|  | Cayman Brac North Outside Lagoon Benthic Sand | 80 | 108 | 80108 |
|  | Cayman Brac North Outside Lagoon Benthic Spur and Groove | 80 | 109 | 80109 |
|  | Cayman Brac North Outside Lagoon Benthic Uncolonized Hardbottom | 80 | 110 | 80110 |
|  | Cayman Brac North Lagoon Benthic Backreef | 80 | 111 | 80111 |
|  | Cayman Brac North Lagoon Benthic Beach Rock | 80 | 112 | 80112 |
|  | Cayman Brac North Lagoon Benthic Hardbottom | 80 | 113 | 80113 |
|  | Cayman Brac North Lagoon Benthic Lagoonal Coral | 80 | 114 | 80114 |
|  | Cayman Brac North Lagoon Benthic Mud | 80 | 115 | 80115 |
|  | Cayman Brac North Lagoon Benthic Seagrass Beds | 80 | 116 | 80116 |
|  | Cayman Brac North Lagoon Benthic Sediment | 80 | 117 | 80117 |
|  | Cayman Brac North Lagoon Benthic Vegetated Sand | 80 | 118 | 80118 |
|  | Cayman Brac North Shoreline Cliff | 80 | 119 | 80119 |
|  | Cayman Brac North Shoreline Cobble | 80 | 120 | 80120 |
|  | Cayman Brac North Shoreline Cobble Mosaic | 80 | 121 | 80121 |
|  | Cayman Brac North Shoreline Ironshore | 80 | 122 | 80122 |
|  | Cayman Brac North Shoreline Ironshore Mosaic | 80 | 123 | 80123 |
|  | Cayman Brac North Shoreline Rock | 80 | 124 | 80124 |
|  | Cayman Brac North Shoreline Rock Mosaic | 80 | 125 | 80125 |
|  | Cayman Brac North Shoreline Sand | 80 | 126 | 80126 |
|  | Cayman Brac North Shoreline Sand Mosaic | 80 | 127 | 80127 |
|  | Cayman Brac North Wetland Mangrove | 80 | 128 | 80128 |
|  | Cayman Brac North Protected Land Crown Land | 80 | 129 | 80129 |
|  | Cayman Brac North Species Main Turtle Nesting Beaches | 80 | 139 | 80139 |
|  | Cayman Brac South Outside Lagoon Benthic Aggregate Reef | 90 | 101 | 90101 |
|  | Cayman Brac South Outside Lagoon Benthic Aggregated Patch Reef | 90 | 102 | 90102 |
|  | Cayman Brac South Outside Lagoon Benthic Beach Rock | 90 | 103 | 90103 |
|  | Cayman Brac South Outside Lagoon Benthic Colonized Hardbottom | 90 | 104 | 90104 |
|  | Cayman Brac South Outside Lagoon Benthic Individual Patch Reef | 90 | 105 | 90105 |
|  | Cayman Brac South Outside Lagoon Benthic Reef Crest | 90 | 106 | 90106 |
|  | Cayman Brac South Outside Lagoon Benthic Rubble | 90 | 107 | 90107 |
|  | Cayman Brac South Outside Lagoon Benthic Sand | 90 | 108 | 90108 |
|  | Cayman Brac South Outside Lagoon Benthic Spur and Groove | 90 | 109 | 90109 |
|  | Cayman Brac South Outside Lagoon Benthic Uncolonized Hardbottom | 90 | 110 | 90110 |
|  | Cayman Brac South Lagoon Benthic Backreef | 90 | 111 | 90111 |
|  | Cayman Brac South Lagoon Benthic Beach Rock | 90 | 112 | 90112 |
|  | Cayman Brac South Lagoon Benthic Hardbottom | 90 | 113 | 90113 |
|  | Cayman Brac South Lagoon Benthic Lagoonal Coral | 90 | 114 | 90114 |
|  | Cayman Brac South Lagoon Benthic Mud | 90 | 115 | 90115 |
|  | Cayman Brac South Lagoon Benthic Seagrass Beds | 90 | 116 | 90116 |
|  | Cayman Brac South Lagoon Benthic Sediment | 90 | 117 | 90117 |
|  | Cayman Brac South Lagoon Benthic Vegetated Sand | 90 | 118 | 90118 |
|  | Cayman Brac South Shoreline Cliff | 90 | 119 | 90119 |
|  | Cayman Brac South Shoreline Cobble | 90 | 120 | 90120 |
|  | Cayman Brac South Shoreline Cobble Mosaic | 90 | 121 | 90121 |
|  | Cayman Brac South Shoreline Ironshore | 90 | 122 | 90122 |
|  | Cayman Brac South Shoreline Ironshore Mosaic | 90 | 123 | 90123 |
|  | Cayman Brac South Shoreline Rock | 90 | 124 | 90124 |
|  | Cayman Brac South Shoreline Rock Mosaic | 90 | 125 | 90125 |
|  | Cayman Brac South Shoreline Sand | 90 | 126 | 90126 |
|  | Cayman Brac South Shoreline Sand Mosaic | 90 | 127 | 90127 |
|  | Cayman Brac South Wetland Mangrove | 90 | 128 | 90128 |
|  | Cayman Brac South Protected Land Crown Land | 90 | 129 | 90129 |
|  | Cayman Brac South Species Main Turtle Nesting Beaches | 90 | 139 | 90139 |

Appendix 2 Final output file of Marxan best scenario for ‘base goals’ and strict status layers locked in.

| Feature ID | Feature Name | Target Amount (acres or miles) | Target Amount (hectares or km) | Amount Held (acres or miles) | Amount Held (hectares or km) | Occurrences Held | Target Met | MPM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 70126 | Little Cayman Shoreline Sand South | 0.678 | 0.274 | 0.700 | 0.283 | 7 | yes | 1 |
| 60126 | Little Cayman Shoreline Sand North | 0.09 | 0.036 | 0.300 | 0.121 | 3 | yes | 1 |
| 70127 | Little Cayman Shoreline Sand Mosaic South | 5.004 | 2.025 | 5.040 | 2.040 | 36 | yes | 1 |
| 60127 | Little Cayman Shoreline Sand Mosaic North | 4.848 | 1.962 | 6.130 | 2.481 | 49 | yes | 1 |
| 60124 | Little Cayman Shoreline Rock North | 0.087 | 0.035 | 0.130 | 0.053 | 1 | yes | 1 |
| 70125 | Little Cayman Shoreline Rock Mosaic South | 3.141 | 1.271 | 3.430 | 1.388 | 15 | yes | 1 |
| 60125 | Little Cayman Shoreline Rock Mosaic North | 1.893 | 0.766 | 1.990 | 0.805 | 20 | yes | 1 |
| 70122 | Little Cayman Shoreline Ironshore South | 1.041 | 0.421 | 1.260 | 0.510 | 8 | yes | 1 |
| 60122 | Little Cayman Shoreline Ironshore North | 5.067 | 2.051 | 5.110 | 2.068 | 13 | yes | 1 |
| 70123 | Little Cayman Shoreline Ironshore Mosaic South | 1.632 | 0.660 | 1.660 | 0.672 | 5 | yes | 1 |
| 60123 | Little Cayman Shoreline Ironshore Mosaic North | 5.43 | 2.197 | 5.440 | 2.201 | 24 | yes | 1 |
| 70120 | Little Cayman Shoreline Cobble South | 0.099 | 0.040 | 0.180 | 0.073 | 2 | yes | 1 |
| 60120 | Little Cayman Shoreline Cobble North | 1.722 | 0.697 | 1.740 | 0.704 | 22 | yes | 1 |
| 70121 | Little Cayman Shoreline Cobble Mosaic South | 2.922 | 1.182 | 2.990 | 1.210 | 30 | yes | 1 |
| 60121 | Little Cayman Shoreline Cobble Mosaic North | 1.107 | 0.448 | 1.280 | 0.518 | 17 | yes | 1 |
| 20126 | Grand Cayman Shoreline Sand West Side | 11.154 | 4.514 | 11.290 | 4.569 | 72 | yes | 1 |
| 30126 | Grand Cayman Shoreline Sand South Side | 7.848 | 3.176 | 7.860 | 3.181 | 80 | yes | 1 |
| 10126 | Grand Cayman Shoreline Sand North Sound | 6.276 | 2.540 | 6.280 | 2.541 | 67 | yes | 1 |
| 40126 | Grand Cayman Shoreline Sand North Side | 5.607 | 2.269 | 5.610 | 2.270 | 56 | yes | 1 |
| 30127 | Grand Cayman Shoreline Sand Mosaic South Side | 0.546 | 0.221 | 0.560 | 0.227 | 7 | yes | 1 |
| 10127 | Grand Cayman Shoreline Sand Mosaic North Sound | 0.468 | 0.189 | 0.470 | 0.190 | 12 | yes | 1 |
| 40127 | Grand Cayman Shoreline Sand Mosaic North Side | 0.048 | 0.019 | 0.050 | 0.020 | 2 | yes | 1 |
| 50127 | Grand Cayman Shoreline Sand Mosaic East End | 0.243 | 0.098 | 0.250 | 0.101 | 4 | yes | 1 |
| 50126 | Grand Cayman Shoreline Sand East End | 3.186 | 1.289 | 3.230 | 1.307 | 36 | yes | 1 |
| 30124 | Grand Cayman Shoreline Rock South Side | 0.927 | 0.375 | 0.950 | 0.384 | 6 | yes | 1 |
| 40124 | Grand Cayman Shoreline Rock North Side | 0.387 | 0.157 | 0.910 | 0.368 | 5 | yes | 1 |
| 20125 | Grand Cayman Shoreline Rock Mosaic West Side | 0.66 | 0.267 | 0.730 | 0.295 | 9 | yes | 1 |
| 30125 | Grand Cayman Shoreline Rock Mosaic South Side | 2.652 | 1.073 | 2.700 | 1.093 | 27 | yes | 1 |
| 10125 | Grand Cayman Shoreline Rock Mosaic North Sound | 0.525 | 0.212 | 0.580 | 0.235 | 6 | yes | 1 |
| 40125 | Grand Cayman Shoreline Rock Mosaic North Side | 2.895 | 1.172 | 2.910 | 1.178 | 30 | yes | 1 |
| 50125 | Grand Cayman Shoreline Rock Mosaic East End | 0.336 | 0.136 | 0.340 | 0.138 | 8 | yes | 1 |
| 50124 | Grand Cayman Shoreline Rock East End | 0.858 | 0.347 | 1.180 | 0.478 | 7 | yes | 1 |
| 20122 | Grand Cayman Shoreline Ironshore West Side | 9.447 | 3.823 | 9.600 | 3.885 | 68 | yes | 1 |
| 30122 | Grand Cayman Shoreline Ironshore South Side | 9.348 | 3.783 | 9.370 | 3.792 | 57 | yes | 1 |
| 10122 | Grand Cayman Shoreline Ironshore North Sound | 3.762 | 1.522 | 3.770 | 1.526 | 27 | yes | 1 |
| 40122 | Grand Cayman Shoreline Ironshore North Side | 0.687 | 0.278 | 0.690 | 0.279 | 8 | yes | 1 |
| 20123 | Grand Cayman Shoreline Ironshore Mosaic West Side | 2.817 | 1.140 | 2.820 | 1.141 | 21 | yes | 1 |
| 30123 | Grand Cayman Shoreline Ironshore Mosaic South Side | 0.081 | 0.033 | 0.160 | 0.065 | 1 | yes | 1 |
| 10123 | Grand Cayman Shoreline Ironshore Mosaic North Sound | 1.53 | 0.619 | 1.540 | 0.623 | 8 | yes | 1 |
| 40123 | Grand Cayman Shoreline Ironshore Mosaic North Side | 1.164 | 0.471 | 1.210 | 0.490 | 5 | yes | 1 |
| 50122 | Grand Cayman Shoreline Ironshore East End | 0.546 | 0.221 | 0.560 | 0.227 | 9 | yes | 1 |
| 30120 | Grand Cayman Shoreline Cobble South Side | 0.228 | 0.092 | 0.350 | 0.142 | 8 | yes | 1 |
| 10120 | Grand Cayman Shoreline Cobble North Sound | 0.045 | 0.018 | 0.060 | 0.024 | 2 | yes | 1 |
| 40120 | Grand Cayman Shoreline Cobble North Side | 0.984 | 0.398 | 1.010 | 0.409 | 7 | yes | 1 |
| 30121 | Grand Cayman Shoreline Cobble Mosaic South Side | 0.558 | 0.226 | 0.640 | 0.259 | 12 | yes | 1 |
| 10121 | Grand Cayman Shoreline Cobble Mosaic North Sound | 0.339 | 0.137 | 0.360 | 0.146 | 4 | yes | 1 |
| 40121 | Grand Cayman Shoreline Cobble Mosaic North Side | 1.083 | 0.438 | 1.100 | 0.445 | 14 | yes | 1 |
| 50120 | Grand Cayman Shoreline Cobble East End | 0.015 | 0.006 | 0.040 | 0.016 | 1 | yes | 1 |
| 30119 | Grand Cayman Shoreline Cliff South Side | 1.32 | 0.534 | 1.730 | 0.700 | 12 | yes | 1 |
| 40119 | Grand Cayman Shoreline Cliff North Side | 0.516 | 0.209 | 0.520 | 0.210 | 10 | yes | 1 |
| 90126 | Cayman Brac Shoreline Sand South | 1.518 | 0.614 | 1.530 | 0.619 | 16 | yes | 1 |
| 80126 | Cayman Brac Shoreline Sand North | 0.051 | 0.021 | 0.120 | 0.049 | 1 | yes | 1 |
| 90127 | Cayman Brac Shoreline Sand Mosaic South | 1.848 | 0.748 | 1.870 | 0.757 | 18 | yes | 1 |
| 80127 | Cayman Brac Shoreline Sand Mosaic North | 1.41 | 0.571 | 1.490 | 0.603 | 13 | yes | 1 |
| 90125 | Cayman Brac Shoreline Rock Mosaic South | 1.425 | 0.577 | 1.610 | 0.652 | 12 | yes | 1 |
| 80125 | Cayman Brac Shoreline Rock Mosaic North | 4.557 | 1.844 | 4.560 | 1.845 | 32 | yes | 1 |
| 90122 | Cayman Brac Shoreline Ironshore South | 1.92 | 0.777 | 2.130 | 0.862 | 8 | yes | 1 |
| 80122 | Cayman Brac Shoreline Ironshore North | 7.77 | 3.144 | 7.780 | 3.148 | 27 | yes | 1 |
| 90123 | Cayman Brac Shoreline Ironshore Mosaic South | 27.198 | 11.007 | 27.220 | 11.016 | 91 | yes | 1 |
| 80123 | Cayman Brac Shoreline Ironshore Mosaic North | 14.172 | 5.735 | 14.200 | 5.747 | 44 | yes | 1 |
| 90120 | Cayman Brac Shoreline Cobble South | 1.173 | 0.475 | 1.180 | 0.478 | 10 | yes | 1 |
| 80120 | Cayman Brac Shoreline Cobble North | 2.13 | 0.862 | 2.200 | 0.890 | 19 | yes | 1 |
| 90121 | Cayman Brac Shoreline Cobble Mosaic South | 1.74 | 0.704 | 1.780 | 0.720 | 13 | yes | 1 |
| 80121 | Cayman Brac Shoreline Cobble Mosaic North | 2.955 | 1.196 | 2.960 | 1.198 | 25 | yes | 1 |
| 90119 | Cayman Brac Shoreline Cliff South | 1.752 | 0.709 | 1.790 | 0.724 | 17 | yes | 1 |
| 80119 | Cayman Brac Shoreline Cliff North | 0.831 | 0.336 | 0.960 | 0.388 | 11 | yes | 1 |
| 70110 | Little Cayman South Benthic Habitat Uncolonized Hardbottom | 191.583 | 77.531 | 222.780 | 90.156 | 373 | yes | 1 |
| 70109 | Little Cayman South Benthic Habitat Spur and Groove | 576.384 | 233.254 | 577.370 | 233.653 | 671 | yes | 1 |
| 70107 | Little Cayman South Benthic Habitat Rubble | 56.118 | 22.710 | 75.070 | 30.380 | 203 | yes | 1 |
| 70106 | Little Cayman South Benthic Habitat Reef Crest | 68.676 | 27.792 | 68.750 | 27.822 | 203 | yes | 1 |
| 70104 | Little Cayman South Benthic Habitat Colonized Hardbottom | 101.308 | 40.998 | 103.760 | 41.990 | 237 | yes | 1 |
| 70102 | Little Cayman South Benthic Habitat Aggregated Patch Reef | 3.92 | 1.586 | 5.280 | 2.137 | 14 | yes | 1 |
| 60110 | Little Cayman North Benthic Habitat Uncolonized Hardbottom | 225.216 | 91.142 | 415.700 | 168.228 | 591 | yes | 1 |
| 60109 | Little Cayman North Benthic Habitat Spur and Groove | 329.756 | 133.448 | 329.970 | 133.534 | 433 | yes | 1 |
| 60108 | Little Cayman North Benthic Habitat Sand | 2.829 | 1.145 | 9.180 | 3.715 | 30 | yes | 1 |
| 60107 | Little Cayman North Benthic Habitat Rubble | 28.377 | 11.484 | 34.270 | 13.869 | 165 | yes | 1 |
| 60106 | Little Cayman North Benthic Habitat Reef Crest | 37.092 | 15.011 | 37.850 | 15.317 | 143 | yes | 1 |
| 60104 | Little Cayman North Benthic Habitat Colonized Hardbottom | 95.92 | 38.817 | 114.810 | 46.462 | 283 | yes | 1 |
| 60103 | Little Cayman North Benthic Habitat Beach Rock | 0.087 | 0.035 | 0.210 | 0.085 | 3 | yes | 1 |
| 60101 | Little Cayman North Benthic Habitat Aggregate Reef | 3.656 | 1.480 | 6.910 | 2.796 | 48 | yes | 1 |
| 20110 | Grand Cayman Benthic Habitat West Side Uncolonized Hardbottom | 312.642 | 126.522 | 943.010 | 381.623 | 1343 | yes | 1 |
| 20109 | Grand Cayman Benthic Habitat West Side Spur and Groove | 377.056 | 152.589 | 838.370 | 339.276 | 1036 | yes | 1 |
| 20108 | Grand Cayman Benthic Habitat West Side Sand | 50.769 | 20.545 | 114.900 | 46.498 | 302 | yes | 1 |
| 20107 | Grand Cayman Benthic Habitat West Side Rubble | 39.063 | 15.808 | 109.040 | 44.127 | 258 | yes | 1 |
| 20105 | Grand Cayman Benthic Habitat West Side Individual Patch Reef | 0.336 | 0.136 | 0.350 | 0.142 | 5 | yes | 1 |
| 20104 | Grand Cayman Benthic Habitat West Side Colonized Hardbottom | 101.296 | 40.993 | 233.700 | 94.575 | 573 | yes | 1 |
| 20103 | Grand Cayman Benthic Habitat West Side Beach Rock | 1.905 | 0.771 | 5.270 | 2.133 | 34 | yes | 1 |
| 20102 | Grand Cayman Benthic Habitat West Side Aggregated Patch Reef | 12.848 | 5.199 | 25.990 | 10.518 | 117 | yes | 1 |
| 20101 | Grand Cayman Benthic Habitat West Side Aggregate Reef | 4.744 | 1.920 | 11.860 | 4.800 | 35 | yes | 1 |
| 30110 | Grand Cayman Benthic Habitat South Side Uncolonized Hardbottom | 334.371 | 135.315 | 364.510 | 147.512 | 615 | yes | 1 |
| 30109 | Grand Cayman Benthic Habitat South Side Spur and Groove | 880.916 | 356.494 | 880.920 | 356.496 | 1081 | yes | 1 |
| 30108 | Grand Cayman Benthic Habitat South Side Sand | 6.864 | 2.778 | 11.660 | 4.719 | 19 | yes | 1 |
| 30107 | Grand Cayman Benthic Habitat South Side Rubble | 100.077 | 40.500 | 140.530 | 56.870 | 327 | yes | 1 |
| 30106 | Grand Cayman Benthic Habitat South Side Reef Crest | 84.336 | 34.130 | 85.330 | 34.532 | 268 | yes | 1 |
| 30104 | Grand Cayman Benthic Habitat South Side Colonized Hardbottom | 70.516 | 28.537 | 98.630 | 39.914 | 167 | yes | 1 |
| 30101 | Grand Cayman Benthic Habitat South Side Aggregate Reef | 15.9 | 6.435 | 15.970 | 6.463 | 28 | yes | 1 |
| 10110 | Grand Cayman Benthic Habitat North Sound Uncolonized Hardbottom | 232.821 | 94.219 | 577.930 | 233.880 | 845 | yes | 1 |
| 10109 | Grand Cayman Benthic Habitat North Sound Spur and Groove | 271.976 | 110.065 | 272.160 | 110.139 | 376 | yes | 1 |
| 10108 | Grand Cayman Benthic Habitat North Sound Sand | 5.001 | 2.024 | 5.350 | 2.165 | 16 | yes | 1 |
| 10107 | Grand Cayman Benthic Habitat North Sound Rubble | 50.283 | 20.349 | 130.110 | 52.654 | 300 | yes | 1 |
| 10106 | Grand Cayman Benthic Habitat North Sound Reef Crest | 31.12 | 12.594 | 64.080 | 25.932 | 242 | yes | 1 |
| 10105 | Grand Cayman Benthic Habitat North Sound Individual Patch Reef | 0 | 0.000 | 0.000 | 0.000 | 1 |  | 1 |
| 10104 | Grand Cayman Benthic Habitat North Sound Colonized Hardbottom | 126.108 | 51.034 | 193.150 | 78.165 | 398 | yes | 1 |
| 10102 | Grand Cayman Benthic Habitat North Sound Aggregated Patch Reef | 0.24 | 0.097 | 0.410 | 0.166 | 3 | yes | 1 |
| 40110 | Grand Cayman Benthic Habitat North Side Uncolonized Hardbottom | 164.58 | 66.603 | 209.670 | 84.850 | 360 | yes | 1 |
| 40109 | Grand Cayman Benthic Habitat North Side Spur and Groove | 251.636 | 101.833 | 251.760 | 101.884 | 349 | yes | 1 |
| 40107 | Grand Cayman Benthic Habitat North Side Rubble | 24.717 | 10.003 | 35.640 | 14.423 | 165 | yes | 1 |
| 40106 | Grand Cayman Benthic Habitat North Side Reef Crest | 31.94 | 12.926 | 32.030 | 12.962 | 144 | yes | 1 |
| 40104 | Grand Cayman Benthic Habitat North Side Colonized Hardbottom | 136.256 | 55.141 | 148.380 | 60.047 | 300 | yes | 1 |
| 40102 | Grand Cayman Benthic Habitat North Side Aggregated Patch Reef | 1.756 | 0.711 | 3.120 | 1.263 | 13 | yes | 1 |
| 40101 | Grand Cayman Benthic Habitat North Side Aggregate Reef | 31.944 | 12.927 | 32.310 | 13.075 | 80 | yes | 1 |
| 50110 | Grand Cayman Benthic Habitat East End Uncolonized Hardbottom | 130.179 | 52.682 | 194.570 | 78.740 | 306 | yes | 1 |
| 50109 | Grand Cayman Benthic Habitat East End Spur and Groove | 524.02 | 212.063 | 524.080 | 212.088 | 587 | yes | 1 |
| 50107 | Grand Cayman Benthic Habitat East End Rubble | 37.947 | 15.357 | 50.540 | 20.453 | 140 | yes | 1 |
| 50106 | Grand Cayman Benthic Habitat East End Reef Crest | 51.22 | 20.728 | 51.360 | 20.785 | 144 | yes | 1 |
| 50104 | Grand Cayman Benthic Habitat East End Colonized Hardbottom | 47.524 | 19.232 | 81.440 | 32.958 | 114 | yes | 1 |
| 90110 | Cayman Brac South Benthic Habitat Uncolonized Hardbottom | 224.091 | 90.686 | 345.620 | 139.867 | 557 | yes | 1 |
| 90109 | Cayman Brac South Benthic Habitat Spur and Groove | 623.04 | 252.135 | 623.280 | 252.232 | 762 | yes | 1 |
| 90108 | Cayman Brac South Benthic Habitat Sand | 4.437 | 1.796 | 13.510 | 5.467 | 39 | yes | 1 |
| 90107 | Cayman Brac South Benthic Habitat Rubble | 13.824 | 5.594 | 33.950 | 13.739 | 117 | yes | 1 |
| 90106 | Cayman Brac South Benthic Habitat Reef Crest | 13.744 | 5.562 | 24.920 | 10.085 | 128 | yes | 1 |
| 90105 | Cayman Brac South Benthic Habitat Individual Patch Reef | 0.024 | 0.010 | 0.060 | 0.024 | 1 | yes | 1 |
| 90104 | Cayman Brac South Benthic Habitat Colonized Hardbottom | 100.92 | 40.841 | 101.760 | 41.181 | 271 | yes | 1 |
| 90102 | Cayman Brac South Benthic Habitat Aggregated Patch Reef | 25.2 | 10.198 | 47.180 | 19.093 | 117 | yes | 1 |
| 80110 | Cayman Brac North Benthic Habitat Uncolonized Hardbottom | 224.256 | 90.753 | 224.580 | 90.884 | 374 | yes | 1 |
| 80109 | Cayman Brac North Benthic Habitat Spur and Groove | 553.108 | 223.835 | 553.310 | 223.917 | 658 | yes | 1 |
| 80107 | Cayman Brac North Benthic Habitat Rubble | 2.655 | 1.074 | 2.760 | 1.117 | 22 | yes | 1 |
| 80106 | Cayman Brac North Benthic Habitat Reef Crest | 2.044 | 0.827 | 2.410 | 0.975 | 10 | yes | 1 |
| 80104 | Cayman Brac North Benthic Habitat Colonized Hardbottom | 91.396 | 36.987 | 91.410 | 36.992 | 222 | yes | 1 |
| 80102 | Cayman Brac North Benthic Habitat Aggregated Patch Reef | 2.276 | 0.921 | 3.990 | 1.615 | 12 | yes | 1 |
| 70128 | Little Cayman Coastal Mangrove South | 7.372 | 2.983 | 7.390 | 2.991 | 21 | yes | 1 |
| 30128 | Grand Cayman Coastal Mangrove South Side | 7.628 | 3.087 | 7.660 | 3.100 | 23 | yes | 1 |
| 10128 | Grand Cayman Coastal Mangrove North Sound | 652.54 | 264.074 | 972.770 | 393.666 | 1300 | yes | 1 |
| 70118 | Little Cayman Lagoon Benthic South Vegetated Sand | 99.4 | 40.226 | 99.400 | 40.226 | 469 | yes | 1 |
| 70117 | Little Cayman Lagoon Benthic South Sediment | 143.715 | 58.159 | 260.140 | 105.275 | 534 | yes | 1 |
| 70116 | Little Cayman Lagoon Benthic South Seagrass Beds | 146.908 | 59.452 | 150.170 | 60.772 | 507 | yes | 1 |
| 70114 | Little Cayman Lagoon Benthic South Lagoonal Coral | 5.496 | 2.224 | 10.150 | 4.108 | 57 | yes | 1 |
| 70113 | Little Cayman Lagoon Benthic South Hardbottom | 40.452 | 16.370 | 45.380 | 18.365 | 224 | yes | 1 |
| 70112 | Little Cayman Lagoon Benthic South Beach Rock | 2.133 | 0.863 | 4.440 | 1.797 | 22 | yes | 1 |
| 70111 | Little Cayman Lagoon Benthic South Backreef | 49.264 | 19.936 | 49.600 | 20.072 | 177 | yes | 1 |
| 60118 | Little Cayman Lagoon Benthic North Vegetated Sand | 35.08 | 14.196 | 35.220 | 14.253 | 135 | yes | 1 |
| 60117 | Little Cayman Lagoon Benthic North Sediment | 18.516 | 7.493 | 24.840 | 10.052 | 170 | yes | 1 |
| 60116 | Little Cayman Lagoon Benthic North Seagrass Beds | 34.048 | 13.779 | 34.330 | 13.893 | 136 | yes | 1 |
| 60114 | Little Cayman Lagoon Benthic North Lagoonal Coral | 4.636 | 1.876 | 7.550 | 3.055 | 69 | yes | 1 |
| 60113 | Little Cayman Lagoon Benthic North Hardbottom | 52.962 | 21.433 | 86.320 | 34.932 | 241 | yes | 1 |
| 60112 | Little Cayman Lagoon Benthic North Beach Rock | 0.15 | 0.061 | 0.420 | 0.170 | 9 | yes | 1 |
| 60111 | Little Cayman Lagoon Benthic North Backreef | 12.344 | 4.995 | 12.360 | 5.002 | 43 | yes | 1 |
| 30118 | Grand Cayman Lagoon Benthic South Side Vegetated Sand | 126.316 | 51.118 | 126.350 | 51.132 | 624 | yes | 1 |
| 30117 | Grand Cayman Lagoon Benthic South Side Sediment | 196.728 | 79.613 | 244.990 | 99.144 | 680 | yes | 1 |
| 30116 | Grand Cayman Lagoon Benthic South Side Seagrass Beds | 231.304 | 93.605 | 231.450 | 93.664 | 611 | yes | 1 |
| 30115 | Grand Cayman Lagoon Benthic South Side Mud | 2.976 | 1.204 | 8.070 | 3.266 | 32 | yes | 1 |
| 30114 | Grand Cayman Lagoon Benthic South Side Lagoonal Coral | 19.984 | 8.087 | 20.030 | 8.106 | 362 | yes | 1 |
| 30113 | Grand Cayman Lagoon Benthic South Side Hardbottom | 119.235 | 48.253 | 163.770 | 66.275 | 531 | yes | 1 |
| 30112 | Grand Cayman Lagoon Benthic South Side Beach Rock | 1.737 | 0.703 | 2.210 | 0.894 | 24 | yes | 1 |
| 30111 | Grand Cayman Lagoon Benthic South Side Backreef | 43.816 | 17.732 | 51.390 | 20.797 | 201 | yes | 1 |
| 10118 | Grand Cayman Lagoon Benthic North Sound Vegetated Sand | 1322.692 | 535.274 | 1337.980 | 541.461 | 1866 | yes | 1 |
| 10117 | Grand Cayman Lagoon Benthic North Sound Sediment | 290.568 | 117.589 | 612.960 | 248.056 | 1100 | yes | 1 |
| 10116 | Grand Cayman Lagoon Benthic North Sound Seagrass Beds | 5687.568 | 2301.677 | 5687.590 | 2301.686 | 6203 | yes | 1 |
| 10115 | Grand Cayman Lagoon Benthic North Sound Mud | 738.798 | 298.981 | 1746.450 | 706.763 | 2044 | yes | 1 |
| 10114 | Grand Cayman Lagoon Benthic North Sound Lagoonal Coral | 23.648 | 9.570 | 25.880 | 10.473 | 147 | yes | 1 |
| 10113 | Grand Cayman Lagoon Benthic North Sound Hardbottom | 487.014 | 197.088 | 589.900 | 238.724 | 1121 | yes | 1 |
| 10112 | Grand Cayman Lagoon Benthic North Sound Beach Rock | 0.141 | 0.057 | 0.330 | 0.134 | 7 | yes | 1 |
| 10111 | Grand Cayman Lagoon Benthic North Sound Backreef | 21.044 | 8.516 | 40.050 | 16.208 | 179 | yes | 1 |
| 40118 | Grand Cayman Lagoon Benthic North Side Vegetated Sand | 34.02 | 13.767 | 34.210 | 13.844 | 233 | yes | 1 |
| 40117 | Grand Cayman Lagoon Benthic North Side Sediment | 36.453 | 14.752 | 37.530 | 15.188 | 238 | yes | 1 |
| 40116 | Grand Cayman Lagoon Benthic North Side Seagrass Beds | 76.228 | 30.848 | 76.310 | 30.882 | 223 | yes | 1 |
| 40114 | Grand Cayman Lagoon Benthic North Side Lagoonal Coral | 1.356 | 0.549 | 1.380 | 0.558 | 51 | yes | 1 |
| 40113 | Grand Cayman Lagoon Benthic North Side Hardbottom | 19.719 | 7.980 | 26.350 | 10.663 | 189 | yes | 1 |
| 40112 | Grand Cayman Lagoon Benthic North Side Beach Rock | 0.69 | 0.279 | 1.060 | 0.429 | 17 | yes | 1 |
| 40111 | Grand Cayman Lagoon Benthic North Side Backreef | 44.764 | 18.115 | 46.420 | 18.786 | 168 | yes | 1 |
| 50118 | Grand Cayman Lagoon Benthic East End Vegetated Sand | 125.032 | 50.599 | 125.510 | 50.792 | 471 | yes | 1 |
| 50117 | Grand Cayman Lagoon Benthic East End Sediment | 109.605 | 44.356 | 153.930 | 62.293 | 483 | yes | 1 |
| 50116 | Grand Cayman Lagoon Benthic East End Seagrass Beds | 276.192 | 111.771 | 281.690 | 113.996 | 536 | yes | 1 |
| 50114 | Grand Cayman Lagoon Benthic East End Lagoonal Coral | 12.332 | 4.991 | 15.710 | 6.358 | 258 | yes | 1 |
| 50113 | Grand Cayman Lagoon Benthic East End Hardbottom | 42.171 | 17.066 | 52.050 | 21.064 | 255 | yes | 1 |
| 50112 | Grand Cayman Lagoon Benthic East End Beach Rock | 0.753 | 0.305 | 0.760 | 0.308 | 8 | yes | 1 |
| 50111 | Grand Cayman Lagoon Benthic East End Backreef | 18.3 | 7.406 | 18.300 | 7.406 | 86 | yes | 1 |
| 90118 | Cayman Brac Lagoon Benthic South Vegetated Sand | 4.664 | 1.887 | 4.760 | 1.926 | 29 | yes | 1 |
| 90117 | Cayman Brac Lagoon Benthic South Sediment | 4.158 | 1.683 | 4.640 | 1.878 | 44 | yes | 1 |
| 90116 | Cayman Brac Lagoon Benthic South Seagrass Beds | 14.496 | 5.866 | 14.590 | 5.904 | 65 | yes | 1 |
| 90113 | Cayman Brac Lagoon Benthic South Hardbottom | 15.09 | 6.107 | 24.100 | 9.753 | 87 | yes | 1 |
| 90112 | Cayman Brac Lagoon Benthic South Beach Rock | 0.246 | 0.100 | 0.250 | 0.101 | 8 | yes | 1 |
| 90111 | Cayman Brac Lagoon Benthic South Backreef | 13.052 | 5.282 | 20.280 | 8.207 | 82 | yes | 1 |
| 80116 | Cayman Brac Lagoon Benthic North Seagrass Beds | 0.032 | 0.013 | 0.050 | 0.020 | 4 | yes | 1 |
| 80113 | Cayman Brac Lagoon Benthic North Hardbottom | 3.018 | 1.221 | 3.800 | 1.538 | 10 | yes | 1 |
| 80111 | Cayman Brac Lagoon Benthic North Backreef | 0.908 | 0.367 | 1.080 | 0.437 | 7 | yes | 1 |
| 70129 | Little Cayman Crown Land South | 3.332 | 1.348 | 3.980 | 1.611 | 8 | yes | 1 |
| 60129 | Little Cayman Crown Land North | 0.636 | 0.257 | 0.710 | 0.287 | 6 | yes | 1 |
| 10129 | Grand Cayman Crown Land North Sound | 233.776 | 94.606 | 486.540 | 196.896 | 679 | yes | 1 |
| 90129 | Cayman Brac Crown Land South | 3.064 | 1.240 | 3.120 | 1.263 | 18 | yes | 1 |
| 80129 | Cayman Brac Crown Land North | 0.412 | 0.167 | 0.500 | 0.202 | 9 | yes | 1 |
| 90139 | Cayman Brac Main Turtle Nesting Beaches South (Line) | 0.684 | 1.101 | 0.700 | 1.127 | 21 | yes | 1 |
| 80139 | Cayman Brac Main Turtle Nesting Beaches North (Line) | 0.184 | 0.296 | 0.280 | 0.451 | 8 | yes | 1 |
| 70139 | Little Cayman Main Turtle Nesting Beaches South (Line) | 1.496 | 2.408 | 1.510 | 2.430 | 50 | yes | 1 |
| 60139 | Little Cayman Main Turtle Nesting Beaches North (Line) | 1.764 | 2.839 | 1.770 | 2.849 | 51 | yes | 1 |
| 50139 | Grand Cayman Main Turtle Nesting Beaches East End (Line) | 0.136 | 0.219 | 0.200 | 0.322 | 6 | yes | 1 |
| 40139 | Grand Cayman Main Turtle Nesting Beaches North Side (Line) | 1.396 | 2.247 | 1.400 | 2.253 | 49 | yes | 1 |
| 30139 | Grand Cayman Main Turtle Nesting Beaches South Side (Line) | 1.504 | 2.420 | 1.700 | 2.736 | 50 | yes | 1 |
| 20139 | Grand Cayman Main Turtle Nesting Beaches West Side (Line) | 2.144 | 3.450 | 2.480 | 3.991 | 78 | yes | 1 |
| 10139 | Grand Cayman Main Turtle Nesting Beaches North Sound (Line) | 1.164 | 1.873 | 1.310 | 2.108 | 42 | yes | 1 |

Appendix 3 Data (2011) used for the Simplified Integrated Reef Health Index for the Cayman Islands.

| Site ID | Site Name | Key Commercial Fish (g.100 m2) NB: Grouper & Snapper only | Value | Key Herbivorous Fish (g.100m2) NB: parrot fish and Surgeonfish only | Value | Mean % Live Hard Coral Cover | Value | Mean Fleshy Macroalgal Cover (%) | Value | Total | Average (1-5) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Big Tunnels | 6577 | 5 | 1867 | 2 | 12.80 | 3.00 | 31.20 | 1.00 | 11.00 | 2.75 |
| 2 | North West Point (Wpt # 58) | no data |  | no data |  | 5.50 | 2.00 | 11.40 | 3.00 | 5.00 | 2.50 |
| 3 | Shallow Control West Side | 1723 | 5 | 3741 | 5 | 11.30 | 3.00 | 34.65 | 1.00 | 14.00 | 3.50 |
| 4 | Victoria Secret | 845 | 3 | 2990 | 4 | 17.45 | 3.00 | 26.25 | 1.00 | 11.00 | 2.75 |
| 5 | Deep Control | 2030 | 5 | 2398 | 3 | 8.75 | 2.00 | 28.00 | 1.00 | 11.00 | 2.75 |
| 6 | Oro Verde | 5599 | 5 | 2933 | 4 | 10.80 | 3.00 | 26.30 | 1.00 | 13.00 | 3.25 |
| 7 | Holiday Inn Drop-off | 1745 | 5 | 1248 | 2 | 12.10 | 3.00 | 32.30 | 1.00 | 11.00 | 2.75 |
| 8 | Caribbean Paradise | 1420 | 4 | 2109 | 3 | 8.95 | 2.00 | 34.50 | 1.00 | 10.00 | 2.50 |
| 9 | Mesa | no data |  | no data |  | 11.30 | 3.00 | 33.00 | 1.00 | 4.00 | 2.00 |
| 10 | Seaview Reef | 514 | 2 | 1858 | 2 | 17.40 | 3.00 | 20.85 | 2.00 | 9.00 | 2.25 |
| 11 | Eagle Ray Rock | 2036 | 5 | 1295 | 2 | 17.50 | 3.00 | 29.90 | 1.00 | 11.00 | 2.75 |
| 12 | Pallas Reef West | 173 | 1 | 1702 | 2 | 8.75 | 2.00 | 24.25 | 2.00 | 7.00 | 1.75 |
| 13 | Pallas Pinnacle Deep | 2128 | 5 | 1216 | 2 | 10.90 | 3.00 | 67.20 | 1.00 | 11.00 | 2.75 |
| 14 | Spotts Reef | 1031 | 3 | 4199 | 5 | 16.95 | 3.00 | 44.45 | 1.00 | 12.00 | 3.00 |
| 15 | Spotts Deep | 3398 | 5 | 2361 | 3 | 3.55 | 1.00 | 53.45 | 1.00 | 10.00 | 2.50 |
| 16 | Mance Road Shallow | no data |  | no data |  | 13.45 | 3.00 | 28.50 | 1.00 | 4.00 | 2.00 |
| 17 | Mance Road Deep | no data |  | no data |  | 11.05 | 3.00 | 53.15 | 1.00 | 4.00 | 2.00 |
| 18 | Breakers Shallow | 721 | 2 | 2523 | 3 | 11.45 | 3.00 | 40.90 | 1.00 | 9.00 | 2.25 |
| 19 | Breakers Deep | 148 | 1 | 2545 | 3 | 10.50 | 3.00 | 35.00 | 1.00 | 8.00 | 2.00 |
| 20 | McKenny's Canyon | 1612 | 4 | 2053 | 3 | 11.50 | 3.00 | 44.60 | 1.00 | 11.00 | 2.75 |
| 21 | Grouper Grotto | 526 | 2 | 3046 | 4 | 10.60 | 3.00 | 19.50 | 2.00 | 11.00 | 2.75 |
| 22 | Black Rock | 1698 | 5 | 4283 | 5 | 10.55 | 3.00 | 60.85 | 1.00 | 14.00 | 3.50 |
| 23 | Fish Tank | 674 | 2 | 4060 | 5 | 8.50 | 2.00 | 55.25 | 1.00 | 10.00 | 2.50 |
| 24 | Apollo Deep | 168 | 1 | 2590 | 3 | 7.75 | 2.00 | 44.65 | 1.00 | 7.00 | 1.75 |
| 25 | Apollo Shallow | 69 | 1 | 1417 | 2 | 13.05 | 3.00 | 62.00 | 1.00 | 7.00 | 1.75 |
| 26 | Delilah's Delight | 307 | 1 | 2002 | 3 | 8.95 | 2.00 | 39.25 | 1.00 | 7.00 | 1.75 |
| 27 | Andy's Reef | 1670 | 4 | 3049 | 4 | 10.70 | 3.00 | 27.65 | 1.00 | 12.00 | 3.00 |
| 28 | White Stroke Canyon | 3524 | 5 | 2594 | 3 | 5.85 | 2.00 | 54.50 | 1.00 | 11.00 | 2.75 |
| 29 | Pinnacles | 693 | 2 | 1413 | 2 | 4.75 | 1.00 | 47.75 | 1.00 | 6.00 | 1.50 |
| 30 | Main Street | 2621 | 5 | 2496 | 3 | 7.85 | 2.00 | 50.90 | 1.00 | 11.00 | 2.75 |
| 31 | Conch Point / Wpt# 23 / Pappagallo's reef | 353 | 1 | 1603 | 2 | 13.25 | 3.00 | 51.85 | 1.00 | 7.00 | 1.75 |
| 32 | Pirates Point Reef | 4457 | 5 | 2916 | 4 | 17.25 | 3.00 | 44.40 | 1.00 | 13.00 | 3.25 |
| 33 | Paddy's Place | 3433 | 5 | 2528 | 3 | 25.13 | 4.00 | 54.87 | 1.00 | 13.00 | 3.25 |
| 34 | Black Hole | 4630 | 5 | 1826 | 2 | 30.10 | 4.00 | 35.75 | 1.00 | 12.00 | 3.00 |
| 35 | Windsock Reef | 3292 | 5 | 3439 | 4 | 26.45 | 4.00 | 42.35 | 1.00 | 14.00 | 3.50 |
| 36 | Lucas Ledges | 397 | 1 | 1654 | 2 | 17.05 | 3.00 | 47.00 | 1.00 | 7.00 | 1.75 |
| 37 | Rock House Reef | 2799 | 5 | 2646 | 3 | 10.93 | 3.00 | 51.47 | 1.00 | 12.00 | 3.00 |
| 38 | Coral City | 1028 | 3 | 2464 | 3 | 19.15 | 3.00 | 40.25 | 1.00 | 10.00 | 2.50 |
| 39 | Diggary Point | 836 | 2 | 1897 | 2 | 12.60 | 3.00 | 39.67 | 1.00 | 8.00 | 2.00 |
| 40 | Sail Fin Reef | 211 | 1 | 1021 | 2 | 10.55 | 3.00 | 36.75 | 1.00 | 7.00 | 1.75 |
| 41 | Booby Pass | 704 | 2 | 1295 | 2 | 9.53 | 2.00 | 19.33 | 2.00 | 8.00 | 2.00 |
| 42 | Waypoint 9 | 507 | 2 | 1180 | 2 | 10.70 | 3.00 | 25.85 | 1.00 | 8.00 | 2.00 |
| 43 | Waypoint 20 | 143 | 1 | 1538 | 2 | 12.65 | 3.00 | 31.25 | 1.00 | 7.00 | 1.75 |
| 44 | Eagle Ray Roundup | 934 | 3 | 1308 | 2 | 17.40 | 3.00 | 23.00 | 2.00 | 10.00 | 2.50 |
| 45 | Bus Stop / Jackson Reef and Wall | 4913 | 5 | 2056 | 3 | 12.85 | 3.00 | 28.10 | 1.00 | 12.00 | 3.00 |
| 46 | Three Fathoms Wall | 5318 | 5 | 2036 | 3 | 13.25 | 3.00 | 31.20 | 1.00 | 12.00 | 3.00 |
| 47 | Randy's Gazebo | 2731 | 5 | 2647 | 3 | 8.10 | 2.00 | 34.25 | 1.00 | 11.00 | 2.75 |
| 48 | Tarpon Reef | 1369 | 4 | 1350 | 2 | 7.60 | 2.00 | 45.25 | 1.00 | 9.00 | 2.25 |
| 49 | Butterfly Reef | 91 | 1 | 2078 | 3 | 13.35 | 3.00 | 46.90 | 1.00 | 8.00 | 2.00 |
| 50 | Seafeather Wall | 1870 | 5 | 1004 | 2 | 22.40 | 4.00 | 52.00 | 1.00 | 12.00 | 3.00 |
| 51 | Cayman Cays | 509 | 2 | 1477 | 2 | 18.50 | 3.00 | 45.45 | 1.00 | 8.00 | 2.00 |
| 52 | Manta canyon | no data |  | no data |  | 20.07 | 4.00 | 58.47 | 1.00 | 5.00 | 2.50 |
| 53 | Waypoint 42 | 221 | 1 | 1550 | 2 | 14.20 | 3.00 | 56.93 | 1.00 | 7.00 | 1.75 |
| 54 | Waypoint 41 | 176 | 1 | 1786 | 2 | 13.10 | 3.00 | 29.35 | 1.00 | 7.00 | 1.75 |
| 55 | Rock Monster Chimney | no data |  | no data |  | 18.50 | 3.00 | 51.85 | 1.00 | 4.00 | 2.00 |
| 56 | Bert's Brother | 298 | 1 | 1623 | 2 | 10.65 | 3.00 | 72.35 | 1.00 | 7.00 | 1.75 |
| 57 | Tailor Wall West | no data |  | no data |  | 7.07 | 2.00 | 36.53 | 1.00 | 3.00 | 1.50 |
| 58 | Shark Hole | 1089 | 3 | 1098 | 2 | 16.20 | 3.00 | 43.75 | 1.00 | 9.00 | 2.25 |
| 59 | Jan's Drop | 191 | 1 | 1038 | 2 | 6.60 | 2.00 | 63.93 | 1.00 | 6.00 | 1.50 |
| 60 | Grunt Valley | 3532 | 5 | 1609 | 2 | 9.45 | 2.00 | 58.40 | 1.00 | 10.00 | 2.50 |
| 61 | Charlie's Reef | 1846 | 5 | 1756 | 2 | 8.10 | 2.00 | 37.00 | 1.00 | 10.00 | 2.50 |
| 62 | Cemetery Wall | 1149 | 3 | 1461 | 2 | 12.70 | 3.00 | 47.40 | 1.00 | 9.00 | 2.25 |
| 63 | Piper's wall | no data |  | no data |  | 6.00 | 2.00 | 57.15 | 1.00 | 3.00 | 1.50 |